65C02 Instruction Set Extensions

by David Empson (dempson@actrix.gen.nz)

All opcodes are given in hex.  Columns are Opcode, Bytes, Cycles and Syntax.

**********  Changes to existing instructions and CPU behaviour

JMP (abs)

On the 6502, a JMP indirect with the pointer at location xxFF (i.e. the

last byte of a page) will fetch the high byte of the jump address from

the first byte of the same page (xx00).  On the 65C02, the high byte is

fetched from the first byte of the next page, as expected ((xx+1)00).

The 6502 takes 5 cycles for this instruction, the 65C02 takes 6.  It is

not clear from the documentation I have here whether the extra cycle is

added in all cases or only when a page boundary is crossed.

ADC all modes

SBC all modes

In decimal mode, the 6502 does not affect the N, V and Z flags when these

instructions are used.  These flags are affected on the 65C02.  The 65C02

required one extra cycle for ADC/SBC when decimal mode is enabled.

Indexing over a page boundary

On the 6502, this will perform an extra read of an invalid address (the

destination address minus 0100).  On the 65C02, the last instruction byte

is re-read during this cycle.

Invalid opcodes

On the 6502, many invalid opcodes do undocumented operations.  Some cause

the processor to hang until reset.  On the 65C02, all invalid opcodes are

treated as NOPs with varying numbers of cycles.

Read/modify/write instructions

The 6502 performs one read and two writes, with the unmodified data on the

first write.  The 65C02 performs two reads and one write.

Decimal flag

On the 6502, this flag is in an undefined state after reset and not changed

on interrupt.  On the 65C02, D is always clear after reset or interrupt.

Interrupt after fetch of BRK instruction

On the 6502, the interrupt will be handled and the BRK is forgotten.  On

the 65C02, the BRK is handled, then the interrupt is handled.

**********  New addressing modes for existing instructions

Op  By  Cy  Syntax

12  2   5   ORA (zp)

32  2   5   AND (zp)

52  2   5   EOR (zp)

72  2   5   ADC (zp)

92  2   5   STA (zp)

B2  2   5   LDA (zp)

D2  2   5   CMP (zp)

F2  2   5   SBC (zp)

All of the above behave like XXX (zp),Y or XXX (zp,X) with the index

register containing zero, i.e. indirect without indexing.

Op  By  Cy  Syntax

1A  1   2   INC A

3A  1   2   DEC A

Obvious extensions.  Some assemblers use alternative notations such as

"INA" and "DEA", or "INC" and "DEC" without an operand.

Op  By  Cy  Syntax

34  2   4   BIT zp,x

3C  3   4*  BIT abs,X       Add 1 cycle if indexing over page boundary

89  2   2   BIT #imm

The usual BIT operation.  Note that bits 6 and 7 are NOT affected by the

BIT IMMEDIATE instruction.

Op  By  Cy  Syntax

7C  3   6   JMP (abs,X)

Used for jumping through tables.

**********  New Instructions

Op  By  Cy  Syntax

80  2   3*  BRA address     Add 1 cycle if page boundary crossed

Unconditional relative branch.

Op  By  Cy  Syntax

DA  1   3   PHX

5A  1   3   PHY

FA  1   4   PLX

7A  1   4   PLY

Push/pull index registers.  Flags are affected as for PHA/PLA.

Op  By  Cy  Syntax

64  2   3   STZ zp

74  2   4   STZ zp,X

9C  3   4   STZ abs

9E  3   5   STZ abs,X

Store zero at destination address.  No registers or flags are affected.

Op  By  Cy  Syntax

14  2   5   TRB zp

1C  3   6   TRB abs

04  2   5   TSB zp

0C  3   6   TSB abs

Test and reset/set bits.  This is a read/modify/write instruction which

combines the BIT operation with a memory set/clear.

1. The accumulator and the memory location are ANDed.  The result is used

to set the zero flag but is not stored anywhere.

2. Bits in the memory location are set (TSB) or reset (TRB) according to the

mask in the accumulator.  Each "1" bit in A causes the corresponding

memory bit to be set or reset.  The accumulator is not affected.

**********  Rockwell R65C02

The Rockwell R65C02 has several additional instructions which are not present

in other 65C02s.  These instructions are used to set, reset and test/branch on

specific bits in zero page locations.  They use all the x7 and xF opcodes.

These instructions are not compatible with the 65802/65816, which use these

opcodes for new instructions and addressing modes.

The instructions are as follows.  If you support these, I suggest that they

have to be enabled by a special directive (or macros could be used).

Op  By  Cy  Syntax          Op  By  Cy  Syntax

07  2   5   RMB0 zp         0F  3   5*  BBR0 zp,addr

17  2   5   RMB1 zp         1F  3   5*  BBR1 zp,addr

27  2   5   RMB2 zp         2F  3   5*  BBR2 zp,addr

37  2   5   RMB3 zp         3F  3   5*  BBR3 zp,addr

47  2   5   RMB4 zp         4F  3   5*  BBR4 zp,addr

57  2   5   RMB5 zp         5F  3   5*  BBR5 zp,addr

67  2   5   RMB6 zp         6F  3   5*  BBR6 zp,addr

77  2   5   RMB7 zp         7F  3   5*  BBR7 zp,addr

87  2   5   SMB0 zp         8F  3   5*  BBS0 zp,addr

97  2   5   SMB1 zp         9F  3   5*  BBS1 zp,addr

A7  2   5   SMB2 zp         AF  3   5*  BBS2 zp,addr

B7  2   5   SMB3 zp         BF  3   5*  BBS3 zp,addr

C7  2   5   SMB4 zp         CF  3   5*  BBS4 zp,addr

D7  2   5   SMB5 zp         DF  3   5*  BBS5 zp,addr

E7  2   5   SMB6 zp         EF  3   5*  BBS6 zp,addr

F7  2   5   SMB7 zp         FF  3   5*  BBS7 zp,addr

*   BBRn/BBSn   Add 1 cycle if branch occurs to same page, 2 cycles if

branch occurs to different page.

**********  Assembler Notes

Most assemblers that support the 65C02 have a directive to enable the 65C02

instructions.  This directive may default to OFF or ON.  If OFF, the assembler

should only accept standard 6502 instructions, and will report some kind of

error for a 65C02 instruction.

For example, the ORCA/M assembler on the Apple IIgs supports the 6502, 65C02

and 65816.  It defaults to 65816 mode (native for the IIgs), and has the

following directives:

65816 OFF

65816 ON

65C02 OFF

65C02 ON

--

David Empson

dempson@actrix.gen.nz

Snail mail: P.O. Box 27-103, Wellington, New Zealand