Documentation for a2asm

=======================

OVERVIEW

a2asm is an assembler for the 65c02 microprocessor. The object file

is in a "proprietary" format which is similar to "a.out" format.

COPYRIGHT

a2asm is Copyright (c) 1995 by Archie L. Cobbs. However, you

are free to copy, modify, and distribute it under the terms

spelled out in the file COPYING.

INVOCATION

If the -6502 flag is given, only 6502 instructions are allowed.

If the -l flag is given, an assembly listing is sent to standard

output.

LINE SYNTAX

The syntax and stuff is based loosely on the Merlin assembler syntax.

Sectioning pseudo-ops, etc. are based on "convention".

SECTIONING

The assembler allows assembly into three sections, or segments.

These are CODE, DATA, and BSS. The latter is for uninitialized

data. Therefore, when BSS is active only non-emitting statements

are allowed in your assembly file. The BSS section is not actually

stored in the object file, only it's length. By convention, before

a program is executed the BSS section is allocated and zeroed.

There is no ".org" directive; the code origin is determined at

load time (using a2load).

COMMENTS

Anything after a semi-colon is a comment and is ignored.

If an asterisk is the first character on a line, the entire line

is a comment and is ignored.

IDENTIFIERS

Identifiers may contain letters, digits, and underscores, but must

not start with a digit. Identifiers can have arbitrary length.

CONSTANTS

Character constants are surrounded by single quotes. The high bit

of the byte is determined by the setting of the .hibit pseudo-op,

except in the case of hexadecimal and octal escapes. The escape

sequences, valid in both character and string constants, are:

\a   - Bell ("alert" -- $07)

\n   - Newline ($0A)

\r   - Carriage return ($0D)

\b   - Backspace

\e   - Escape

\v   - Vertical tab

\t   - Horizontal tab

\f   - Formfeed

\NNN - Octal value (0 <= N <= 7)

\xNN - Hexadecimal value (0 <= N <= F)

\'   - Single quote

\"   - Double quote

\\   - Backslash

String constants are surrounded by double quotes, and may be

concatentated.

Integer constants are as decimal, hexadecimal (preceeded by

'$' or '0x'), octal (preceeded by '0'), or binary (preceeded

by '%'). They may also be preceeded by '+' or '-'.

Floating constants are in the usual formats, for example:

42

1.0

3.141

-.443e12

The ".float" pseudo-op generates 5 byte AppleSoft style floating

points quantities. This format is described in "float.doc".

Floating point constants are not allowed in expressions, since

expressions operate on integer values only. An integer expression

in a ".float" pseudo-op is computed as an integer, then converted

to floating point. So the instruction

.float  5/2

generates the floating point value 2.0.

LINE FORMAT

The line syntax is pretty much standard, something like this:

[label] [:] [opcode] [argument [, argument ... ] ]

Fields are separated by whitespace, i.e., spaces, tabs, etc.

No whitespace is allowed before the label, if there is one.

See the file "a2asm.bnf" for a more precise description.

EXPRESSIONS

Expression format follows the C language very closely, so if you

are familiar with C you should feel at home. If not, _The C Programming

Language_ by Kernighan & Ritchie is a good reference. The one major

exception is that square brackets are used for grouping expressions

rather than parentheses.

The usual C precedence and grouping is respected. In order of precedence:

-----------------------------------------------------

Category                Operator        Associativity

-----------------------------------------------------

unary                   ! ~ -           right to left

binary (multiplication) * / %           left to right

binary (addition)       + -             left to right

binary (shift)          << >>           left to right

binary (relational)     < <= > >=       left to right

binary (equality)       == !=           left to right

binary (bitand)         &               left to right

binary (bitxor)         ^               left to right

binary (bitor)          |               left to right

binary (logical AND)    &&              left to right

binary (logical OR)     ||              left to right

conditional             ?:              right to left

Notes:

- The ~ operator computes one's complement.

- The ! operator computes logical negation.

- Conditionals work like this:

x ? y : z

returns y if x != 0, otherwise z.

- Right shift is either arithmetic or logical, depending

on what your host's C compiler does in the same situation.

There is one major exception. Because a parenthesized expression like

jmp  ( label + 4 )

could be interpreted as either an indirect jump or as a jump to the

value of a parenthesized expression, we use square brackets for

expression grouping and leave parentheses for indirection and indexing.

Therefore the above statement is an indirect jump; to get the other

meaning, use

jmp  [ label + 4 ]

General operands can be any expression allowed in the language C.

However, if you do anything besides adding or subtracting from a

symbol, it's value must be an absolute constant known at assembly

time. In other words, if the result can't be fit into the object file

format, there's no way the assembler can assemble it.

The operand * returns the address (and section) of the current line.

PROCESSOR INSTRUCTIONS

Instructions may be in upper or lower case, but not a mixture.

a2asm does not require/allow an argument to an implied mode instruction.

Therefore, to do an ASL on the accumulator, you just say:

asl

with no argument. a2asm accepts a few synonyms for the same instruction:

xor = eor

blt = bcc

bge = bcs

INSTRUCTION ARGUMENTS

Indexed and indirect addressing modes use parentheses, e.g.:

lda (zp),y

sta (zp,x)

jmp (abs)

To indicate the high byte or low byte of an expression:

lda #expr       ; Low byte of expr

lda #<expr      ; Low byte of expr

lda #>expr      ; High byte of expr

PSEUDO-OP INSTRUCTIONS

=       - Set label equal to given value

.equ    - Synonym for =

.assert - Assembly fails unless argument is non-zero

.align  - Align next address to a multiple of argument

.ascii  - Store ASCII string data

.asciz  - Store ASCII string followed by a zero byte

.dci    - Like ASCII, but reverse the high bit of the last byte

.code   - Assemble into the CODE section

.data   - Assemble into the DATA section

.bss    - Assemble into the BSS section; only ".space" allowed

.byte   - Store low order bytes of given list of values

.dbyt   - Store list of values as big endian 16 bit quantities

.long   - Store list as little endian 32 bit quantities

.int    - Synonym for .long

.float  - Emit a 5 byte AppleSoft style floating point value

.word   - Store list as little endian 16 bit quantities

.zero   - Fill this many bytes with zero

.space  - Allocate this many bytes; negative values are allowed

.ds     - Synonym for .space

.hibit  - Characters and strings get their hi bits set if non-zero

.expand - Verbose assembly listing if non-zero

.export - Make symbols in the list known to other object files

.global - Synonym for .export

.import - Assume symbols in list are defined in other object files

.extern - Synonym for .import

.zimport- Like .import, but assume values are less than $100

PREPROCESSING

You may use the normal C preprocessor on your input file first,

allowing #ifdef, #include, and so on. The assembler understands

#line directives, so errors in included files are reported correctly.

CONDITIONAL ASSEMBLY

Conditional assembly is effected by

.if <constant expression>

.else

.endif

If the constant expression is non-zero, the .if part is emitted

and not the .else part; otherwise, it's the reverse. Note that

all the code gets assembled, just some of it doesn't generate

any bytes into the object file.

The .else part may be omitted.

The display of the non-emitted part in the listing is affected

by the setting of .expand.

MACROS

This is not implemented yet.

HOST MACHINE ASSUMPTIONS

It is assumed that sizeof(char *) <= sizeof(unsigned), and that

sizeof(unsigned) == 4. Also, if you use two different architectures

which compile the structures in "include/obj.h" differently (or are

different endian-ness), then the object files will be incompatible.

For ".float" work properly, your host machine must support the IEEE

standard format and sizeof(double) must be 8.

ERRORS

The error reporting is somewhat minimalist. However, with a line-based

language like assembly, it's not a big problem. The worst part is that

sometimes you may get the uninformative "parse error", which just means

that the input line didn't even make it past the parser. Usually this

means the problem is obvious.

If you get a "sync error", you probably imported something .zimport

but not until after it was used.

No error should cause the assembler to crash; if it does, report it

as a bug (see below).

DEFAULTS

The following default settings can be changed by editing the relevant

#define statements in "asm/asm.h":

- The assembler by default allows 65c02 instructions.

- The default output filename is "a.out".

- The initial section for assembly is CODE.

- .expand defaults to ON.

- .hibit defaults to ON.

OBJECT FILE FORMAT

A new format was required to handle the following case: suppose "foo.s"

contains:

.import symbol

lda     #>symbol

Then the object file needs a relocation entry for the immediate operand

of the "lda" instruction. Specifically, it needs bits 8-15 of the ultimate

value of "symbol". Most object file formats don't handle this case of an

"extracted" byte.

We allow simple offsets from external symbols, nothing fancier. So you

can't say, for example:

.import symbol

.word   symbol * 2

But you can say:

.import sybol

.word   symbol + 2

See the file "include/obj.h" for details on the format. Don't be fooled

by the section AbsSegment; this is just where constant values "live".

BUGS

There are probably several. Please let me know about any you find,

even if it's not a really a bug but just a bothersome feature.

Thanks. Email to: archie@cs.berkeley.edu.