This document describes the JSPICE3 Josephson junction model.

I. derivation of the model.

The expression for the junction current is

J = C*dV/dT + G(V) + Ic(Ci)*sin(P),	   (1)

where:

J      is the total device current

C      is the capacitance

V      is the junction voltage

G(V)   is the quasiparticle conductance

Ci     is the control current

Ic(Ci) is the supercurrent amplitude

P      is the junction phase

The integration method is either trapezoidal or rectangular.  Thus for a new

time point for some variable X being integrated,

Trap:  X  = X0 + d*(X' + X0')/2		   (2)

X' = -X0' + 2*(X - X0)/d		   (3)

Rect:  X  = X0 + d*X'			   (4)

X' = (X - X0)/d			   (5)

Where:

X    is the value at the present time point

X0   is the value at the previous time point

X'   is the time derivative at the present time point

X0'  is the derivative at the previous time point

d    is delta time

Thus, the first term in (1) can be expressed

C*dV/dT ->  C*(-V0' + 2*(V - V0)/d)    [Trap]  (6)

C*(V - V0)/d               [Rect]  (7)

In the matrix formulation, one solves AV = I, where A is the coefficient

operator matrix, V is the node voltage vector, and I the source current

vector.  To linearize the problem, one linearizes A, and modifies I with

the residue terms.

For the capacitance term, the matrix element is the term which multiplies V.

ELT = 2*C/d  [Trap]	 (8)

ELT = C/d    [Rect]	 (9)

The RHS vector is modified as

RHS = RHS + C*(V0' + 2*V0/d)	 [Trap]  (10)

RHS = RHS + C*V0/d            [Rect]  (11)

The other terms, being generally nonlinear, are solved using Newton

iterations.  This is done by linearizing the terms around the previous

iteration result.  For the second term, one can write

G(V) ~ G(VL) + (V - VL)*g	(12)

where:

V     is the current voltage accros the device

G(V)	 is the present nonlinear current through the device

VL    is the voltage calculated by the last Newton iteration

G(VL) is the current at the last iteration

g     is the derivative dG/dV evaluated at VL

Thus,

ELT = g			 (13)

RHS = RHS - G(VL) + VL*g	 (14)

The supercurrent term is slightly more complicated because there are two

independent variables.

Ic(Ci)*sin(P) ~ (Ic(CiL) + (Ci-CiL)*Ic')*(sin(PL) + (P-PL)*cos(PL)) (15)

Neglecting terms second order in the difference,

Ic(Ci)*sin(P) ~

(Ic(CiL) + (Ci-CiL)*Ic')*sin(PL) + Ic(CiL)*(P-PL)*cos(PL)       (16)

The phase is given by dP/dt = V/Cp, where Cp is a constant.  Thus, using

(2-5) above,

P = P0 + d*(V + V0)/(2*Cp)     [Trap]	(17)

P = P0 + d*V/Cp                [Rect]	(18)

This holds for the last Newton intertion as well

PL = P0 + d*(VL + V0)/(2*Cp)   [Trap]	(19)

PL = P0 + d*VL/Cp              [Rect]	(20)

Thus

P - PL = d*(V - VL)/(2*Cp)   [Trap]	(21)

= d*(V - VL)/Cp       [Rect]	(22)

Rewriting (16)

Ic(Ci)*sin(P) ~

(Ic(CiL) + (Ci-CiL)*Ic')*sin(PL) +

Ic(CiL)*cos(PL)*(V-VL)*d/(2*Cp)  [Trap]  (23)

(Ic(CiL) + (Ci-CiL)*Ic')*sin(PL) +

Ic(CiL)*cos(PL)*(V-VL)*d/Cp	  [Rect]  (24)

There are two matrix elememts:

ELTvv = Ic(CiL)*cos(PL)*d/(2*Cp)  [Trap]  (25)

ELTvv = Ic(CiL)*cos(PL)*d/Cp      [Rect]  (26)

ELTvi = Ic'*sin(PL)

The RHS term is modified accordingly

RHS = RHS - Ic(CiL)*sin(PL) + CiL*Ic'*sin(PL)

+ Ic(CiL)*cos(PL)*VL*d/(2*Cp)	  [Trap]  (27)

RHS = RHS - Ic(CiL)*sin(PL) + CiL*Ic'*sin(PL)

+ Ic(CiL)*cos(PL)*VL*d/Cp	  [Rect]  (28)

Define:

crt  = Ic(CiL)*sin(PL)

dcrt	= Ic'*sin(PL)

ag   = 2/d if Trap, 1/d if Rect

gcs  = Ic(CiL)*cos(PL)/(ag*Cp)

ELTvv = gcs

ELTvi = dcrt

RHS   = RHS - crt + CiL*dcrt + VL*gcs