------------------------------------------------------------------------------

|                                                                              |

|                                                                              |

|                                                                              |

|                  An Introduction to LOG with AnaLOG                          |

|                           5.62                                        |

|    LOG and DigLOG, Copyright (C) 1985, 1990 by David Gillespie               |

|    AnaLOG, Copyright (C) 1985, 1990 by John Lazzaro                          |

|    Correspondence to lazzaro@cs.berkeley.edu or daveg@synaptics.com          |

|                                                                              |

------------------------------------------------------------------------------

_______________________________________________________________________________

Table of Contents

General information

Digital simulator

Analog simulator

Other simulation features

Plotting circuits and data

LOG-to-NTK conversion

LOG-to-SPICE conversion

Creating your own gates

Reference material

_______________________________________________________________________________

General Information

Getting started

Using the program

Circuit editing

Configuring gates

More about editing

LOG survival tips

GETTING STARTED

1.  LOG is a large circuit editing and simulation system.  It has

facilities for digital simulation (the original LOG), analog

simulation (AnaLOG), network generation (LOGNTK), and plotting

(LPLOT).  You probably won't need to use all of these, so large

parts of this document may not be relevant to you.  This

document was originally written to describe AnaLOG, so it is

biased toward that application. Another learning aid for anaLOG

are the lessons files located in the /usr/chipmunk/log/lib directory.

2.  To run LOG, give the command:

% diglog   or

% analog

This runs LOG and sets it up for digital or analog simulation.

Both simulators are present in both of these files; the only

difference is which gates you get in your default menu.

3.  To exit at any time, use the Exit command from the menus.

4.  To report bugs, send e-mail to:

lazzaro@cs.berkeley.edu

or  daveg@synaptics.com

USING THE PROGRAM

1.  You control LOG mainly with the mouse, and also from the

keyboard.  It takes some time to get comfortable with the

mouse in LOG, but it will soon become second nature.  There are

three basic mouse motions you need to learn:

2.  TAP.  Press and release the left button on a spot.  This should

feel like tapping a key on a keyboard.  The spot you are tapping

is indicated by the cursor on the screen, usually a small

arrow.

3.  PRESS/DRAG.  Press down and hold the left button, move the cursor

somewhere else, and release.

4.  REST.  Press the right button and release.  This is used to cancel

operations like drawing wires.

5.  Some LOG functions appear right on the screen, so you can

activate them just by tapping the word.  Others are in "menus,"

which you get by pressing on one of the words Frills,

Editing, Cursor, or Misc, then dragging to and releasing

on the word you want.  If you decide you didn't want the menu

after all, just release on some other part of the screen.

6.  There are also keyboard commands, though everything you really

need as a novice can be found in on-screen menus.  When you

press a key for LOG, be sure the cursor is in one of the LOG

windows (it doesn't matter which one).  There are several

features of the keyboard to notice:

7.  SINGLE KEYS.  Many commands appear on keys as well as menus,

just for convenience.  For example, you can press the letter

"s" to do the same thing as SCOPE in the Misc menu.

8.  ":" COMMANDS.  You can give full-word commands by pressing the

":" key, then typing the command.  This is sometimes more

convenient; for example, if you know the name of the file you

want to load, typing ":LOAD FOOBAR" is faster than selecting

LOAD from Misc, then pointing to the file name.  [NOTE: The

Unix version of LOG does not support this feature yet.]

9.  <CNTRL>-C. Press the Control key and the "C" key together

to exit from any mode you may find yourself in.

SURVIVAL TIP #1:  If you get lost and confused, press <CNTRL>-C

until you know where you are.

10.  SPACE BAR.  The space bar at most times means to refresh the

screen.  If the screen has gotten trashed by too much editing,

press this key to clean it up.  LOG will also silently clean

up the screen if you stop editing for a few seconds.

CIRCUIT EDITING

1.  At the bottom of the screen are several circuit symbols.  In

LOG lingo, these are called gates.  The area above the blue

lines is the drawing area, where you draw your circuit

diagrams.  When you start AnaLOG, two very special gates, the

the clock icon and the AnaLOG "scoreboard," show up automatically.

You are welcome to move these around or delete them if you like,

but you will find them extremely handy to have nearby.  (If you

are using LOG as a digital simulator these gates will not appear.)

2.  To begin drawing a circuit, choose the circuit symbol you want,

press down on it and drag it (still holding down)

to the desired place.  To move a gate, just press and drag.

To delete a single gate, just "throw it away" by moving it to

the bottom corners of the screen.

3.  If you don't see the gate you want in the menus, tap the word

CAT to see the "gate catalog."  You can then press and drag any

of these gates to add it to your circuit.  If you will be using

a lot of a particular kind of gate, you can drop it into one of

the several menu slots to replace another less interesting gate.

If you get into the catalog by accident, press an empty part of

the screen, or, as usual, hit <CNTRL>-C to get out.

4.  If you look closely at a gate that has been put into the drawing

area, you will see some red dots.  These are called the gate's

red dots, or pins, and they are the places to connect other

circuit elements.  The easiest way to connect two gates together

is to put them down with the appropriate red dots on top of each

other.

SURVIVAL TIP #2:  The single most common mistake in LOG is to

stick two gates, e.g. a capacitor and a ground,

so close together that they look connected but

their red dots pass through each other.  The gates

are NOT connected, and the circuit will NOT simulate

correctly.  The solution is to nudge the gates

apart a bit, then refresh the screen (space bar)

to make sure the red dots are aligned.

5.  If the gate comes in an awkward orientation, you can tap it to

rotate it by 90 degrees.  This works for gates in the circuit

as well as for menu-area gates.  Notice that LOG distinguishes

between tapping a gate, to rotate it, and pressing a gate, to

move it.  Practice a few times until you can tap and press at

will, without getting confused.  (Some gates, like switches,

do special things when you tap them in the circuit.  These can

only be rotated in the menu area.)

6.  A much neater way to connect things is with wires.  To draw a

wire, just tap the place you want to start, move horizontally

or vertically, then tap the endpoint.  This automatically starts

a new wire at the endpoint, so you can make a chain of connected

wires.  To finish the chain, just tap the right mouse button.  The

whole sequence of tap-tap-rest takes a few minutes to learn but

soon feels very natural.

7.  You can move wires by pressing and dragging, just like gates.

If you press in the middle of the wire, you move it from side

to side but keep the same length.  If you press one of the ends,

you also get to stretch that end.  As usual, to delete a wire

just grab it and drag to the edge of the screen.

8.  Wires are connective along their whole lengths, so you can be

sloppy about connecting to gates as long as you at least overlap

the red dot.  Wires and gates are drawn in slightly different

colors so you can tell them apart if you need to.

9.  If you connect wires in a "T", LOG "solders" them automatically.

If you cross them in a +, LOG assumes they are unconnected, so

you must go back and tap the intersection to solder them.

Because of this, "T"-connections are preferable.

10.  If you should run out of room on the screen for your circuit,

you have several options:  press ">" or "<" to "zoom" in or out

on the circuit (these are also available in the Frills menu),

or use the arrow keys to "scroll" up, down, left, or right.  The

drawing area is essentially infinitely large; so large, in fact,

that you can get lost due to excessive scrolling.  If this

happens, HOME in the Cursor menu will take you back to the

center of the drawing universe.

11.  LOG can keep up to 9 circuit diagram "pages" in memory at a time.

Each page is associated with a different file.  To switch to a

new page, press one of the digit keys 1 through 9.

CONFIGURING GATES

1.  Once you have drawn, say, a capacitor, you may wonder what its

capacitance is, and how to set the capacitance you want.  The

way to do this is called configuration.

2.  In the lower-right corner of the screen is the word ROT with a

curving arrow.  This tells LOG that when you tap a gate, it should

rotate it by 90 degrees.  If you press the word ROT, it changes to

MIR.  Now if you tap a gate it mirrors itself left-to-right.

Pressing MIR leads to MIR with a bar (up-down mirroring), then to

CNFG in yellow, then back to ROT.  If you tap a gate while in CNFG

mode, instead of reorienting it you open it up for configuration.

3.  Each kind of gate has a set of "attributes," usually numbers,

that tell more specifically what kind of circuit element it is.

For example, AnaLOG capacitors have a "capacitance" attribute,

and transistors have a hatfull of attributes about width/length,

kT/q, parasitic capacitances, etc.

4.  In the digital world, most gates do not have attributes.  An

exception is the clock generator, CLOCK, which you can configure to

control the clock frequency and waveform.  Also, many digital gates

have text on their attribute screens that describes how to use the

gate.

5.  One of the attributes is highlighted.  To highlight a different

attribute, press the up- and down-arrow keys.

6.  To enter a new value for an attribute, highlight it, then just

type in the value you desire.

7.  Some attributes can have scale factors, e.g., "m" for milli-

and "u" for micro-.  These will show up as, say, "1.2mV",

meaning 1.2 millivolts.  When you type a new value, you can

always leave off the V since that part is obvious, but don't

forget to include the "m" if that's what you want!  A common

mistake is to change 20pA to 30A, thus seriously perturbing

the operation of the simulated circuit.  When in doubt, you

can always type scientific notation in "1.2E-4" format.

8.  Some attributes are choices instead of numbers.  For example,

voltage sources let you choose a waveform of either "DC,"

"Pulse," or "Sine."  You can type whole words to change these

attributes, or use the arrow keys to cycle through the choices.

9.  To return an attribute to its default value, just press ENTER

all by itself on that line.  Some attributes can be blank, in

which case whatever feature they correspond to is disabled.

Others must have values, and LOG will use some default if you

try to enter "blank."

10.  When a gate is first created, all its attributes have their

default values.  However, if you change a gate's attributes,

then drop that gate into one of the menu area slots, then

new gates drawn from that slot will get those new values

for their attributes.  You can also reconfigure gates in

their menu slots using Probe mode (in the Cursor menu.)

11.  To finish configuring a gate, press the right mouse button again, or

hit <CNTRL>-C.

MORE ABOUT EDITING

1.  In the Editing menu there are commands for doing large-scale

circuit editing that would be unreasonable with just the basic

mouse motions.

2.  DELETE:  Press Editing, drag to Delete, release left mouse button.

Now when you move up into the drawing area, the cursor is shaped

like a pair of scissors.  You can now delete individual things by

touching them (with the point where the blades of the scissors

cross), or you can delete everything in a rectangular region by

pressing at one corner of the region and dragging to the opposite

corner.  Press the right button to exit Delete mode.  You can also

press the "d" key to get the Delete mode scissors. Due to an

intermittent bug, you may need to click the right button a second

time for the scissors to appear.

3.  PASTE:  After you have deleted a group of objects, you can

select Paste to "paste" down copies of what was in that region.

The Paste command highlights the region that was actually

deleted; you can paste back in there for a simple "undelete,"

or you can paste other places for moving or replicating a chunk

of the circuit.  This is also on the "*" key.

4.  COPY:  This is similar to Delete, except that it doesn't actually

delete the contents of the rectangle.  It is for making copies

of something without disturbing the original.  After you have

copied your object or area of objects, you are automatically

switched into Paste mode.  This is also on the "/" key.

5.  MOVE:  Move an area.  This command is like a DELETE followed by

a PASTE; it moves an object, or all the objects in a given area,

to a new location.  It is also on the "m" key.  Note that

the pressing "m" and tapping on a wire moves the wire in a

different (often useful) way than simply grabbing the wire.

6.  Implicit MOVE/COPY:  Pressing in empty space and dragging out a

box automatically does a combination MOVE and COPY command.  The

first place you select moves the contents of the box to that place;

if you press at additional places, extra copies are drawn there.

7.  Open/Close commands:  These are for expert users only, because

using them improperly may damage LOG's internal data structures.

What they do is shift things around on the screen to make extra

room for additions, or to close up excess space.  If you want to

do this, use Delete/Paste instead, or ask someone.

8.  The Labels command in the Frills menu lets you add text labels to

your diagram.  Select Labels, type the label you want, then grab it

and drag it, as usual, into place.  You can edit a label by tapping

it, then using the keyboard.  Labels generally have no effect on the

simulation, but they make plots look much nicer.  Also, certain

tools such as LOGNTK and the Digital LOG hierarchical simulation

gates can read the labels for additional information about the

circuit.

9.  The Boxes command in the Frills menu adds yellow dashed boxes

to the diagram, again for cosmetic purposes only.  Once you're

in Boxes mode, you can drag out boxes in the usual fashion.

To edit boxes, grab a corner for reshaping or an edge for

moving.  Both labels and boxes can be deleted by throwing them

off the bottom corners of the screen.

10.  To save your circuit, select Save in the Misc menu, then type

the file name to use.  It's a good idea to hit Shift-S, or type

":SAVE", periodically to save the circuit under the previous name

if any changes have been made.  ":SAVE" with no file name argument

saves all pages which have been editing back into their original

files.  Circuits are saved in files ending with ".lgf" (for LOG

Format).  The previous version of the file is backed up under the

same name but with ".lfo".

11.  To load a circuit you have saved before, select Load from the Misc

menu.  You will get a list of saved circuits in your directory.  You

can tap one of these to load it, or just type the name.

[NOTE: The UNIX version doesn't support directory listing yet,

you'll need to type the name of the file]. AnaLOG saves the complete

simulation state along with the circuit, but you may want to do a

RESET after loading anyway.

12.  The Grid command in the Cursor menu switches between the

usual arrow cursor and a cross-hair cursor which is useful for

aligning things.

13.  The Yardstick command in the Frills menu lets you draw an

arrow on the screen which you can use to check the geometric

arrangement of objects on the screen.  The yardsticks are not

"real"---they go away when you refresh the screen.  You can also

press the "y" key to get a yardstick.

14.  If you select Status from the Misc menu, you get a set of screens

describing various aspects of the LOG system.  Move from screen to

screen with the arrow keys.  The first screen includes the date and

time, amount of memory free (approx.), plus various other stuff.

Later screens display the status of the LOG editing buffers and

keyboard mappings, then there is a screen for the LPLOT program

(that handles the PLOT command described below), and for the analog

simulator.

SURVIVAL TIPS

SURVIVAL TIP #0:  Select Exit from Misc to exit LOG.

SURVIVAL TIP #1:  If you get lost and confused, press <CNTRL>-C

until you know where you are.

SURVIVAL TIP #3:  The single most common mistake in LOG is to

stick two gates, e.g. a capacitor and a ground,

so close together that they look connected but

their red dots pass through each other.  The gates

are NOT connected, and the circuit will NOT simulate

correctly.  The solution is to nudge the gates

apart a bit, then refresh the screen (space bar)

to make sure the red dots are aligned.

DIGITAL SIMULATION

Basics of digital simulation

A tour of the digital gates

Hierarchical digital simulation

Digital VLSI simulation

BASICS OF DIGITAL SIMULATION

1.  Digital LOG is simulating your circuit constantly, even as

you edit.  Every wire has a "state," corresponding to the

voltage on the wire:

Zero        a wire connected to ground,

One         a wire connected to Vdd,

Weak one    a wire connected to Vdd through a pullup resistor,

None        a wire that is floating (not connected)

2.  You send signals into the circuit using switches, and read

the state of the circuit using displays such as LED's.

3.  If you ever drive a node (wire) with both One and Zero at the

same time, the node starts flashing to indicate a "conflict."

Since this is not an analog circuit simulator, digital LOG can

not represent the correct value of the wire, so it flashes the

wire and picks either One or Zero arbitrarily.

4.  Digital LOG uses a "unit delay" simulation model.  All gates

are considered to have the same delay, whether they are NAND

gates or complex counters.  (The STABILIZE command may change

this behavior for short periods of time.)

A TOUR OF THE DIGITAL GATES

1.  The standard gates in the Digital LOG library provide all

of the logic functions you will need to build a digital

circuit.  If you need a function that is not provided, you

can design it yourself with the LOGED program, or using

digital hierarchy.

2.  All the standard library gates use consistent conventions for

their pins.  For example, when a row of pins carries a binary

number, the most significant bit is on the left or bottom end

of the row.  Signals are active-high unless a "bubble" shows

otherwise.

3.  Input/output devices and controls:

SWITCH     Looks like a small pointed box.  It is a voltage

source that drives either One or Zero onto its

output.  The center lights up black or red to

indicate the state of the switch.  Touch the switch

to change its state.

SWITCH2    A smaller switch suitable for packing into tight

rows.

PULSE      Pulse switch.  Looks like a double-pointed box.

It acts much like SWITCH, except that when you

touch it it goes to One only briefly, then back

to Zero.

LED        Digital display; small square box.  It lights up

according to the value being driven on it:  red for

One (or weak One), black for Zero, and background

gray for undriven wires (None).  It has connection

points all around so you can connect to it from any

direction.  (Note:  This means that placing two

LED's right next to each other will short their

pins together.)

LED2       Structurally the same as LED, but the input pins

are treated a little differently.  In LED, all

eight pins are connected together.  Thus touching

LED's will short each other out.  In LED2, the

eight pins are independent.  If any of them is

driven high, the LED2 glows red.  If any is driven

low, it glows black.  If some drive high and some

low at the same time, the LED2 does not glow.

Since the pins are not internally connected, you

can let LED2's touch and they won't mind.

LED3       This is a miniature LED useful for tightly packed

situations.  Connect to it by running the wire

right through the center.

EDGE       Edge detector.  This is the differently-pointed box.

It lights up red whenever it sees any change on its

input, and only resets to black when you touch it.

KEYPAD     Numeric keypad.  This is a 16-key keypad that generates

a binary number from 0000 to 1111 when you press one

of the 16 keys on the keypad.  The lowest output is

most significant.  There is also a "strobe" output

on the bottom, that generates a pulse each time a

key is pressed.

7SEG       Numeric display.  This is a 7-segment LED display,

which displays the symbols 0 through F for inputs

0000 through 1111.  It has two equivalent sets of

inputs, for your convenience.  The most significant

bit is on the left, and at the bottom.  (The two

sets of pins are equivalent; you may use either.)

ASCKBD     ASCII keyboard.  Analogous to KEYPAD, but with an

alphanumeric keyboard instead which generates

8-bit ASCII codes.  Tap a key to generate its

ASCII code (plus a strobe signal).  Tap SH, then

a key, to shift that key.  CT is the Control key.

Configure the gate and select Key-Codes mode to

generate positional codes for all the keys instead

of ASCII.  This lets you handle Shift and Control

yourself, if you like.

ASCDISP    ASCII terminal display.  Shows 16x64 characters.

To print a character, put its ASCII code on the

data pins and pulse the Strobe pin.  Above Strobe

is the Clear pin, which resets the terminal.  This

terminal understands most standard Chipmunk control

codes such as Carriage Return and Line Feed.  Form

Feed clears the screen.  Certain variations on the

control codes can be had by Configuring the gate.

CLOCK      This is a two- or four-phase clock generator or

oscillator.  By default, it is a two-phase square

wave generator in which the outputs are CLK and

not-CLK; the output changes on every time step

(i.e., once per standard gate delay).  You can

reconfigure it to take several time steps per clock

phase, or to clock according to real, physical time,

or to be a "synchronous" clock which changes as

soon as the rest of the circuit has settled down

from the last change.  Also, you can change the

waveform to four-phase mode, where the outputs are

completely non-overlapping CLK1 and CLK2, each with

25% duty cycle.

BREAK      Breakpoint.  This is a box with an X in it.  This

gate is the circuit equivalent of a software

"breakpoint."  When its inputs see a positive- or

negative-going edge (respectively), it turns the

simulation off so you can examine what was the state

of the system at the instant the transition

occurred.

TIE        Pullup resistor.  This generates a weak One, which

can be overridden without conflict by an open-

collector output pulling down.

TIEGND     Pulldown resistor.  Generates a weak Zero.

4.  Connection gates:

VDD        Power supply.  Provides a constant One signal.

GND        Ground.  Provides a constant Zero signal.

TO/FROM    Terminals.  These are the two arrow-shaped gates

in the Catalog.  They are equivalent except for

appearance.  They are used for putting names on

signals:  To name a signal FOO, connect a terminal

to it, then tap the space next to the arrow and type

FOO.  If there are several terminals in the

circuit with the same names, they are all

electrically connected as if by wires.  TO/FROM

names can be used to pass signals between circuit

pages.  Signal names are also used with the Scope

mode, to be described below.  Dropping a TO/FROM

into the menu area preserves its programmed name;

if the name ends in a number, each fresh gate pulled

from the slot gets a new number.  Predefined signal

names include "Gnd" and "Vdd", equivalent to the

GND and VDD gates, and "Reset", which is normally

grounded but receives a pulse every time you do a

Reset command (in the Misc menu).

5.  Standard digital gates:

AND, OR, NAND, NOR, XOR, XNOR

Standard two-input logic gates.  Also, AND3, NOR4,

etc. are multiple-input gates, and INV is an

inverter.

ANDX, ORX  etc.  These are versions of AND, OR, etc. whose

pictures have been transformed according to

DeMorgan's theorem of logic, which says that an AND

gate is just like an OR gate with its output and all

its inputs inverted.

COMPL      This is a complementary-output buffer.  Its two

outputs provide the input and its complement, with

identical propagation delays.

DNEG, DPOS, JKNEG, JKPOS, TNEG, TPOS

Flip-flops.  Each kind comes in negative- and

positive-edge-triggered flavors.  There is a choice

of D flip-flops (which latch a Data bit on each

clock), or J-K flip-flops (which set, reset, toggle,

or stay the same on each clock), or Toggle flip-

flops (which toggle or stay the same on each

clock).

LATCH      A level-sensitive latch.  When G is One, the output

follows the D input.  When G is Zero, the output

freezes at the last D input value that was followed.

SHIFT      A four-bit shift register.  On a falling clock

transition, it either loads from the parallel inputs

(if M is One), or shifts in the direction of the

arrow (if M is Zero).

SRAM8K     Static RAM or ROM, storing 8K bytes of 8 bits.

Address inputs are on the left (MSB lowest).  Data

input/output pins are at the bottom (MSB on the

the left).  Control pins are Chip Enable (CE), Read,

and Output Enable (OE).  If Read is high and CE and

OE are low, the addressed byte is driven on the Data

pins.  If Read and CE are low (regardless of OE),

the value on the Data pins is stored in the RAM.

Otherwise, the Data pins are undriven and ignored.

(To make a ROM, simply leave Read unconnected.)

Configuring the SRAM8K allows you to examine or

change any byte in the memory, control whether the

memory contents should be recorded in saved circuit

files, and save or load the memory to ".hex" data

files.  SRAM8K has the same unit propagation delay

as all Digital LOG gates---bear in mind that real

memory chips are much slower!

6.  There is also a large number of 7400's series TTL gates to choose

from in the gate library.  These are all intended to be close

models of their true counterparts, but there are no guarantees.

In particular, remember all propagation delays are the same.

7.  A set of gates for simulating digital VLSI transistors exists.

All of these gates begin with "V_" and are described below.

8.  The gates whose names begin with "A_" are for use with the ACTEL

design system.  This is a special box which takes network files

produced by LOGNTK from circuits consisting of "A_" gates, and

programs gate array chips while-U-wait to produce fast, custom

digital chips for prototyping.  Use of the ACTEL system is beyond

the scope of this document.

9.  The gates DIGH, FORCEDRV, and INST1 through INST5 are intended

for use with hierarchical definitions, described in the next

section.

10.  To view the "program" that the simulator uses for a gate, press

the shift-D key, then touch the gate in the circuit diagram.

A small window pops up containing the program.  Press somewhere

not on a gate to exit this mode.  To erase the definitions, just

hit the space bar to refresh the screen.  Notations like "#4"

refer to pin numbers; letters like "E" refer to internal state

variables of the gate.  For further documentation, see the

LOGED manual.

HIERARCHICAL DIGITAL SIMULATION

1.  The Digital LOG simulator is capable of simulating hierarchical

designs.  That is, you can draw a complicated circuit on one

page, then include any number of "instances" of that circuit

on other pages in the form of single LOG gates.  While this

feature is specific to the digital simulator (AnaLOG has no

hierarchy, for example), the LOGNTK network file generator also

understands hierarchy using the same notation and conventions.

2.  The simplest way to build hierarchically is by using the

standard digital "instance" gates, INST1 through INST5.  These

gates look like blocks of various sizes, with connection pins

all along their rims.  They also have little arrows so you can

tell if they have been rotated or mirrored.  (NOTE: There are

also "general-purpose" instance gates GINST1 through GINST5

in the library.  They are not digital gates and the digital

simulator will not understand them!  These gates are for use

only with the LOGNTK program.)

3.  To create a new gate from a circuit diagram, simply put your

diagram into one of LOG's nine editing pages.  Now, take an

INSTn gate of an appropriate size and add it to your diagram.

Connect wires to the gate from the diagram in the same way

you plan to connect the gate "in real life".  For example,

you might have three input wires on the left edge, one output

on the right edge, and a clock input on the bottom edge.

4.  Exact positions don't matter for the instance gates.  They

simply divide your connections into four classes, "top",

"bottom", "left" and "right", and count the wires along

each edge.  When you use the gate in another circuit, you

must make the same number of connections on each edge.  The

order of the wires you connect matters, but not their exact

positions along the edge.

5.  Create a label (by pressing the "l" key) which is the

name you want to give this circuit, in double-quotes: "Joe".

This label can be located anywhere on the page that contains

the definition of "Joe".  (An alternate, older notation is

to make a label of the form:  <name: Joe> .)

6.  Switch to the CNFG (configuring) gate-tapping mode, then

tap the instance gate and enter the same name in the

"Instance of" field: Joe.  Quotes are optional, and

upper/lower case don't matter.  Since this gate has the

same name as the page it is on, it becomes a "template" or

"example" gate.  You now have a complete definition for

your new gate.

7.  To use this gate, create another instance gate on a different

page and configure its "Instance of" attribute to have the

name of the definition you want to use.  You do not need to

use the same size or shape of instance gate as you did before.

As long as you make the right number of connections, it will

work.  Instance gates will be dim until they are "happy", that

is, correctly connected and configured in order to match their

(also correct and complete) definition.

8.  If the generic instance gates are too boring, you can create your

own instance gate, with any picture you like, using the LOGED

program.  The easiest way is to copy a gate like INST1 from the

"log.gate" file (ask your local maintainer) into your own private

gate file, then throw away its picture and draw your own.  You can

use this gate in just the same way as the standard instance gates,

except that typically you will only include as many pins on the gate

as you actually plan to use.  If you want to place pins anywhere

in the gate, you can defeat the generic pin assignment method by

adding the following attribute to the gate's LABEL section:-

label "[is-generic] V No:Generic port assignment?"

The program actually looks for the variant at index 0, so only

offering a single option ensures a constant value.

9.  If you don't like template gates, you can instead use labels

containing "port lists."  This works well only for instance

gates you have drawn yourself.  Pins on a gate are numbered

from 1 to N, the total number of pins.  Include in your

definition, instead of the template or example gate,

a label of the form:  <Port: A B C D>  where "A B C D" is

a list of TO/FROM signal names.  In this example, A will be

associated with pin 1 of the instance gate, B with pin 2,

etc.  The port list must have exactly the right number of

names.  In general, port lists are more awkward to use than

templates.  They exist mostly for historical reasons only.

10.  If you run out of pages you might want to put more than one

definition on a single page.  Simply enclose each definition

in a dashed box (press the "b" key to draw one of those).

When Digital LOG looks for the "Joe" label to find the

definition called Joe, and that label is inside a dashed box,

then it considers only the things on that page which lie inside

the box to be part of that definition.  Since each page can be

scrolled essentially infinitely far, there is plenty of room

for all the definitions you need.

11.  Instances may be nested.  The definition for "A" can include

instance gates for definition "B"---as long as "B" does not

include instances of "A"!  Aside from circular references,

any combination is allowed.  (The template gate is a special

kind of "circular reference" which is excused.)

12.  If you include switches or keypads in your definition, they

will be propagated to all instances.  For example, if you make

a definition with a switch set to "0", then the equivalent

node inside every instance will also be "0".  If you flip the

switch, every instance of that definition will simultaneously

switch along with it.  LED's and other "display" gates have

no effect inside hierarchical definitions.

13.  If the definition includes a label of the form:  <Inert A B C>

then gate types A, B, and C are ignored if they appear in the

definition.  Every definition effectively includes the

command <Inert LED 7SEG>.  The word "Inert" and the names of

the gates are case-insensitive.

14.  Digital hierarchy works by compiling the behavioral descriptions

of all the gates in the definition into one big program which

simulates the instance gates.  Most gates' programs are written

purely in this language, which is described in the documentation

for the LOGED program.  However, some gates, such as the 8K static

RAM, are defined by Pascal procedures instead.  The digital

hierarchy compiler can not compile these gates, and will display

an error message if you try to use one in a definition.  Nearly

every SSI- and MSI-level gate in the standard library is written

purely in LOGED-language, though, so this is not often an issue.

15.  Every time you make any change to the definition, Digital LOG must

"recompile" the definition.  If the definition is large, this can

take time.  While you are editing a definition, you may wish to

turn simulation off (press the "o" key).  No digital gate

definitions will be recompiled until you turn the simulator back

on.

16.  To gain more control over the digital hierarchy compiler, include

a DIGH gate in your definition.  The DIGH gate displays the status

of the compilation of the definition it is in, and also has

controls that affect the kind of compilation that is performed.

You can see the results of the compilation by pressing Shift-D

(the DEFINE command) and tapping on any relevant instance gate

(including the template).

17.  Normally, Digital LOG applies many optimization steps when compiling

a definition which make the resulting instance program simpler.

Depending on what is in the definition, optimization can make the

gate substantially faster to simulate, both by making the overall

simulator faster, and by reducing the propagation delay of the gate.

Optimization may not be safe for definitions that include race

conditions or other effects dependent on exact gate delays; also, it

may be slow for large definitions.  You can configure the DIGH gate

(by tapping it while in CNFG mode, on the blue border of the gate)

to specify optimization only on request, in which case you must

touch the "Optimize" button on the DIGH gate in order to optimize

the definition.

18.  The DIGH gate's configuration screen also has a switch to disable

delay-folding optimizations; with these disabled, gate delay

characteristics of the defining circuit are exactly modeled,

with only simple local optimizations performed.

19.  Another useful optimization for instance programs involves the

FORCEDRV gate, which looks like a little diamond.  Placing this

diamond on any wire in a definition tells the digital instance

compiler to assume this node will always be driven with a "1" or

"0" during operation, that is, it will never be left floating.

You can still leave one of these nodes unconnected if you wish,

but the simulator will no longer guarantee that the results will

be the same as for original definition circuit.  In some cases

the presence of FORCEDRV's allows LOG to create a much more

compact and efficient instance program.  FORCEDRV's are ignored

if the optimizer is turned off.

20.  Configuring DIGH lets you control other aspects of the hierarchy

compiler.  In particular, you can set the "Dump mode" to "Write",

then specify the name of a file.  Then, if you touch "Write" on the

DIGH gate at some later time, the definition will be recompiled and

written into a file which LOGED can read.  This allows you to

convert your definition into a permanent gate in the library.

To configure the DIGH gate, touch around the blue-colored edges of

the gate, away from the buttons in the center of the gate. WARNING:

converting definitions into permanent gates is buggy, and is not

recommended. Expect this bug to be fixed in an upcoming release.

DIGITAL VLSI SIMULATION

1.  The group of gates whose names begin with "V_" are intended to

emulate digital CMOS transistors and structures in the Digital LOG

simulator.  Naturally they are not as accurate a simulation as

AnaLOG's transistors, but they have the advantages that the digital

simulator is much faster, and that they can be used in combination

with all the other digital gates in the library.

2.  The basic VLSI gates are V_NFET and V_PFET, the CMOS transistors.

These gates can model most digital CMOS circuits which do not

involve charge sharing or other "analog" effects.  The transistors'

gate pins have "capacitance" which holds the last charge stored on

them, allowing you to build dynamic as well as static registers.

The transistors are symmetric:  You don't have to worry about which

end is the source and which is the drain.

3.  V_NFET conducts zeros between its source and drain pins whenever its

gate pin is one.  It does not conduct logic ones at all.  Similarly,

V_PFET conducts logic ones when its gate is zero.  In other words,

V_NFET and V_PFET act just like CMOS transistors, as far as most

digital applications are concerned, and that's all you need to know.

If you are interested in a detailed understanding of how the CMOS

simulation works in LOG, read the following few paragraphs.

4.  Because Digital LOG is a gate-level simulator, it needs to know at

any given time which pins are "inputs" and which are "outputs".

Since transistors are symmetrical devices which can conduct in

either direction, the V_NFET and V_PFET gates are implemented as

bidirectional buffers which can change their direction of transfer

according to their environment.  Specifically, any time a V_NFET

sees a zero on its output, and an undriven node on its input, it

changes direction so that it can conduct the zero onto the undriven

node.  This change of direction occurs only if the gate is switched

on.  Likewise, the V_PFET switches direction in order to conduct a

one onto an undriven node.  In most circuits, this all happens

automatically and you can ignore it.  But if a particular V_NFET or

V_PFET looks like it is getting confused, you might swap it for a

V_NFETD or V_PFETD (described below).

5.  The "gate capacitance" is implemented by driving a "weak" one or

zero onto the gate, according to the last "strong" value seen there.

A weak one or zero is the kind of signal produced by a TIE (resistor

to Vdd) or TIEGND (resistor to Gnd) gate.  If another circuit drives

an actual value onto the gate, this strong value overrides the weak

"capacitative" value.  To avoid undesirable charge-sharing problems,

the sources and drains of transistors treat weakly driven ones or

zeros as if they were undriven.  This means, for example, that a

value stored dynamically on the gate of a transistor will not leak

back through any "pass" transistors connected to that gate.

6.  There are also "directional" transistors, V_NFETD and V_PFETD.

These gates are similar to V_NFET and V_PFET, respectively, except

that they conduct in a fixed direction.  Information is always

propagated *from* the pin with the arrow, *to* the other pin.  In

terms of current, the directions of the arrows show the direction of

current flow.

7.  Finally, there are V_NFETX and V_PFETX.  They are directional

transistors that do not "weakly" drive their gates to show the

dynamically stored value there.  Instead, they record the value in

a hidden internal state variable in the transistor.  V_NFETX and

V_PFETX are provided as a minimal-frills transistor, in case the

fancier transistors are too "smart" to work properly in your

circuit.  Also, since the programs for V_NFETX and V_PFETX are

simple they will simulate faster, especially in hierarchical

designs.

8.  V_NFETX and V_PFETX can pass weakly driven signals as well as

strongly driven ones.  As a result, you can use them to simulate

NMOS circuits.  Use V_NFETX for enhancement devices, and the

standard TIE gate as a depletion (pull-up) device.  The fancier

transistors will not work in this way because the the weak one from

the TIE gate will not be able to overpower the weak one or zero

being driven by the transistor's capacitance.

9.  The following additional gates are provided in the VLSI library:

V_BUF      VLSI buffer.  This gate acts like a non-inverting

buffer made out of V_NFETX and V_PFETX transistors.  That

is, it does not affect its input pin in any way, but the

output pin gets a copy of the most recently driven input

value.  You can use V_BUF to get a visible indication of

what value is stored on the hidden gate capacitances of

V_NFETX's and V_PFETX's on the same wire.

V_INV      VLSI inverter.  This gate acts like a usual digital

inverter.  The difference between V_INV and the standard

INV gate is that V_INV's input is dynamic in the same way

as the V_NFET and V_PFET transistors' gates.  V_INV

weakly drives its input according to the capacitively

stored value.  It is provided simply as a shorthand to

drawing the two transistors.

V_AND, V_OR, V_NAND, V_NOR

VLSI AND, OR, NAND, and NOR gates.  These gates have

capacitive inputs, just as in V_INV.

V_TRANS, V_TRANSN

CMOS transmission gates (with active-high or active-low

control).  These gates act much like V_NFET's and

V_PFET's connected together to form the traditional CMOS

"pass" gate.  They require only a single control input,

generating the inverted control signal internally.  For

V_TRANS, the gate conducts ones or zeros in either

direction whenever the control is one, and is an open

circuit when the control is zero.  For V_TRANSN, the gate

conducts only when the control is zero.  If the gates get

confused and you need a directional transmission gate,

you can use the 74125 and 74126 "tri-state buffer" gates

from the library.

V_CSRL     Complementary Set-Reset Logic element.  This is a one-bit

latch designed to emulate a CSRL gate.  If the "latch"

input is high, then a zero on the upper data input clears

the latch and a zero on the lower data input sets the

latch.  If the latch input is low, or if both data inputs

are high or undriven, the latch remembers its previous

value.  The result is undefined if the latch is high and

both data inputs are low.  The latched value and its

complement are driven on the outputs.

V_CSRL0    To get a complete two-phase master-slave flip-flop,

connect two V_CSRL's in series.  The V_CSRL0 gate is

electrically equivalent to V_CSRL, but with the control

pin in a different place so that parallel "phase 1" and

"phase 2" clock busses can be used.

V_CSRL2    Two-bit parallel CSRL latch.

V_CSRL4    Four-bit parallel CSRL latch.

V_CSRLN    CSRL latch with active-low control.

_______________________________________________________________________________

ANALOG SIMULATION

Basics of analog simulation

A tour of the analog gates

Analog input waveforms

BASICS OF ANALOG SIMULATION

1.  LOG and AnaLOG are simulating your circuit constantly, even

as you edit.  But unlike the digital simulator, AnaLOG requires

your circuit to be "complete" before it can simulate.  That is,

every pin must be connected to another pin, not left dangling.

If the circuit is incomplete, i.e., there are still unconnected pins

on some of the gates, AnaLOG will display a message to that effect

on its scoreboard.  As soon as the circuit is complete, the

scoreboard will switch to "Simulation in Progress".

2.  The gate that looks like a box with an arrow on it is the METER.

To monitor your simulation, sprinkle meters liberally around on

all of the interesting voltages.  There is no limit to the

number of meters you can have.  Remember that the meter's red

dot is at the tip of the arrow, so the tip must be exactly

touching the wire or pin of interest.  If the meter is

unconnected or unhappy, it will display a "~" sign.

3.  The "clock icon" is the round thing in the upper-right corner

that looks vaguely like a clock.  Its importance is in the two

numbers directly below it.  These are the current time and

current timestep, respectively.  At the beginning, these are

both zero.  As soon as the simulation begins, the current

time will start advancing.  How fast it is advancing is shown

as the current time-step, and may vary depending on how fast

things are happening in the circuit.

4.  A common error is to watch, say, a capacitor charging, then

go in and double the capacitance, and complain that it still

seems to be charging "at the same rate."  If you look at the

clock icon, though, you will find that the timestep is twice

what it used to be:  AnaLOG can simulate the ramp function

just as quickly in "real time," but the circuit is (correctly)

half as fast in "simulated time."  Make a habit of paying

attention to the clock so that your observations have the

correct frame of reference in time.

5.  Although simulation happens continuously, it is sometimes

necessary to "reset" the circuit back to its starting conditions.

For example, suppose you remove a wire from a running circuit and

the voltages that were in the circuit at the time are "impossible"

for the new circuit.  The simulator will work as hard as it can

to figure out what to do, but may eventually give up and print a

message.  Another example is that you may want to reset various

nodes to the initial voltages that you have programmed for them,

as described in the next paragraph.  In any case, all you have to

do is select RESET in the Misc menu, or press the "R" key.  This

sets the simulation clock back to 0 seconds, and returns the

voltages to their initial values.

6.  If you open any AnaLOG gate for configuration, one thing you will

find is a set of three attributes for every pin on the gate.

These attributes are present voltage, reset voltage, and parasitic

capacitance, respectively.  Every pin has a slight parasitic

capacitance to ground, initially 10fF.  This capacitance must

exist for the simulation to run, but you can set it lower if

you find that it's in your way.  The "present voltage" is an

actual indicator of the voltage currently on that pin, and it

changes while you watch during simulation.  You can enter new

values here if you wish, but the simulator may not like you if you

go around making instantaneous changes in the circuit's voltages.

The "reset voltage" is used by the RESET command.  If you leave

it blank, AnaLOG will solve for an appropriate initial voltage

at RESET time.  Notice that Present and Reset voltages are for

nodes, not gates: if you change a pin's reset voltage, then

open up another gate connected to that same node, the reset

voltage shows up there, too.

7.  When you reset, you will notice that the timestep jumps down,

usually to about 1E-18 seconds, then starts increasing.  This

is partly because AnaLOG is cautiously trying to discover how

fast it can safely proceed in the simulation, and partly

because it has to solve for unspecified initial voltages by

simulating the brief charge-up that would occur when you

powered up a real circuit from scratch.  If you push RESET

and the voltages seem to be frozen, don't despair -- wait for

the timestep to get out of the femtosecond range!

8.  The AnaLOG scoreboard has some words on its left and right

sides.  On the left is Memory, with indicators Set and Erase.

If you tap the word "Erase," all of your "reset voltages" will

be erased.  You might do this if editing has caused your reset

voltages to be no longer appropriate, and you want AnaLOG to

solve the circuit from scratch.  If you tap the word "Set,"

all of the present voltages in the circuit are copied into the

reset voltages.  For example, you might turn your input

waveforms off, let the circuit settle to a stable state, then

push Set so that next time you RESET, the circuit will start

out at that stable state and you won't have to wait again.

9.  AnaLOG is normally very cautious about voltage changes.  If it

is going along at a particular timestep and some node suddenly

starts changing more than about .1V per timestep, AnaLOG will

reduce the timestep and simulate in smaller chunks.  This is

because trying to simulate fast voltage changes all at once

results in ugly and inaccurate waveforms.

10.  Sometimes all you are looking for is a DC point anyway, and

you don't care how ugly your transient waveforms are.  In this

case, you can touch the word "Relaxed" on the scoreboard.  This

tells AnaLOG to take timesteps as large as it can regardless of

how fast the voltages are changing.  Touch "Exact" to return to

an accurate simulation.  Often what people do is touch Relaxed

right after a RESET so that the initial charge-up proceeds

quickly, then touch Exact when the timestep gets up to the

picosecond-nanosecond range.

A TOUR OF THE ANALOG GATES

1.  Clock Icon (TIME).  This displays the current time and timestep.

It also has a square in the center that changes color to indicate

how the simulation is going:

Black       Simulation is turned off

Red         Computing the next timestep

Yellow      A timestep has been computed

If you configure the clock icon you will find that it can

display a few other time-related values, of interest mostly

to AnaLOG implementors only.  Note that TIME is not actually

part of the analog simulator---you can use it for the digital

simulator, too, but it's not very useful there since the

digital simulator has such a crude concept of time.

2.  VDD and GND.  Global power supply.  VDD defaults to +5V.

You can change the value of Vdd in the NUMBERS gate, but

Gnd is always zero, and Vdd is always positive.

3.  Terminals (TO, FROM).  These are the two arrow-shaped gates

in the Catalog.  They are equivalent except for appearance.

They are used for putting names on signals: to name a signal

FOO, connect a terminal to it, then tap the space next to

the arrow and type FOO.  If there are several terminals in

the circuit with the same names, they are all electrically

connected as if by wires.  Signal names are also used with

the Scope mode, to be described below.

4.  Scoreboard (NUMBERS).  This gate serves many purposes.  On

the screen it displays whether simulation is proceeding and

why; it gives controls for set/erase/exact/relaxed modes;

and, when you tap the center bar, it lets you configure

AnaLOG's "global" parameters, such as the value of VDD and

the maximum allowable timestep.  More on these parameters

below.

5.  Transistors. Both three-terminal (NFET7T, PFET7T) and

four-terminal (NFET7F, PFET7F) transistor models are

provided. Only geometric information and parameter

variations are specified on the screens of these gates.

Other gates control the physical constants (PHYSICAL)

device technology parameters (DEVTECHP, DEVTECHN), and

fab-run specific parameter variations (RUNSPEC) of all

transistors in the FET7 series. In addition, the temperature

of all FET7 gates can be varied using the THERMAL gate. For

backward compatability, earlier MOS models are also available

(NFET4, PFET4, NFET5, PFET5, PFET6) as well as FET models

(NSPC1, PSPC1, contributed by Bhusan Gupta) designed to be

used with the SPICE netlist tool (see LOGSPC section). Bipolar

transistor gates and a diode gate (NPN1, PNP1, DIODE) are

experimental Ebers-Moll models. The log/lib/mos.cnf

and log/lib/models.cnf hold default configuration

parameters for transistor models.

6.  Capacitor (CAPFLOAT).  Typical capacitor, defaults to 1pF.

Notice that, like all circuit elements, it has a slight

parasitic capacitance to ground as well as the capacitance

between the two leads.

7.  Resistor (RESFLOAT).  A plain linear resistor.  For a more

VLSI-like resistor, see HRES below.

8.  Transconductance amplifiers (OPAMP, WRAMP).  These are

simulations of the narrow- and wide-range amplifiers in Carver's

book.  Their behavior is modeled after the circuits at the

transistor level (as modeled by AnaLOG), at least if you use

them in the normal range of voltages (between 0 and 5V).  A

little red light turns on if the voltages go outside the range

in which the model is a safe approximation for the "real" circuit.

Note that the "Iset" pin at the bottom wants a voltage, not

a current.  If you want to set the current manually, connect

a voltage source to the pin and also to a standalone transistor,

then measure the current through the transistor.

9.  Rectifiers (HWR, FWR).  These are half-wave and full-wave

rectifiers.  An HWR models an OPAMP driving a two-transistor

current mirror; an FWR models is like a pair of HWR's.

10.  Horizontal resistor (HRES).  This big ugly thing is the

"horizontal resistor" circuit, which predates the circuit in

Carver's book. Obsolete for new designs.

11.  Ganglion (GANGLION).  This is the basic ganglion circuit,

as described in Carver's Book.

12.  Voltage/current meter (MMETER).  This meter displays the voltage

on whatever wire or pin it is connected to.  If you configure it

you can change it to a current meter, in which case it measures

current into the gate from whatever pin it is connected to.

(If you connect a current meter to the middle of a wire, it

doesn't work because the direction of the current is not well-

defined.)

13.  Special magic current meter (ISCOPE).  This is a widget for

use in plotting currents in Scope mode, and will be discussed

in the later section on Scope mode.

14.  Voltage source (VDIFF).  This gate creates a differential

voltage between its two pins (the bottom pin is typically

connected to ground).  It has a finite output resistance

(initially 50 ohms), so it is not an "ideal" source in the

sense of SPICE.  You can use it as a simple DC source with

a certain voltage (set by configuring the gate), or you can

set to output a pulsed or sinusoidal waveform.  This is all

described below, under Input Waveforms.  Voltage sources

display the current-limiting condition by lighting up a

little red light, just like the ones on the lab bench!

They also have a switch, in the lower-right, that you can

switch to freeze the waveform in time.

15.  Current source (IDIFF).  Basically a current-sourcing version

of the voltage source.  It can generate the same kinds of

waveforms.  It voltage-limits if the voltage across the pins

goes below the "crowbar voltage," 0.1V by default.

16.  Step generator (STAIRS).  A voltage source generating a

stairstep function, which moves from one voltage to another

in a certain number of steps, then repeats.

17.  Voltage switch (VSWITCH).  A voltage source that can be

manually switched between "on" and "off" voltage levels.

This one also has a tiny switch on its face, this time to

select which voltage is generated.  It can also be set in

"monostable" mode, in which a tap on the switch will turn

it on for a certain period of time, then back off again

automatically.  Note that voltages here are implicitly

relative to ground.

18.  Current switches (ISWITCH1, ISWITCH2).  Current-sourcing and

current-sinking versions of the VSWITCH.  The current source

(arrow pointing out) is like an IDIFF connected to VDD, and

the current sink (arrow pointing in) is like IDIFF connected

to GND.

19.  Piece-wise linear two-terminal element (PWL, contributed by

Harold Levy). Implements an arbitrary two-terminal element,

with an i-v transfer curve. Cd to the log/lib directory,

start analog, and load the file pwl-test.lgf to learn more.

20.  Resonant tunneling diode (RTD, contributed by Harold Levy).

Models a class of quantum well devices. If you do research

in this area and want more details, contact har@caltech.edu.

ANALOG INPUT WAVEFORMS

1.  DC.  This is just a constant amount of voltage or current.

2.  Pulse.  Depending on how you set it up, this can be a square wave,

triangle, or sawtooth.  Referring to the attributes for Pulse mode,

the output waveform starts at "initial voltage," then, after

"delay time" (plus "reset time" for the first cycle), it begins

ramping to "pulsed voltage," at a rate determined by "rise time."

It stays at that voltage for "pulse width," then ramps back to the

"initial voltage" in "fall time."  The cycle repeats the the period

specified.  NOTES:  If rise and fall time are both relatively

small, you get a square wave.  If one is large and the other small,

you get a sawtooth.  If both are large, and the pulse width is zero,

you get a triangle.

3.  Sine.  This generates a sine wave, with the frequency, offset, and

amplitude desired.  The "phase" is specified in degrees.  If the

"delay" is nonzero, the wave waits that amount of time after RESET

before it begins to change.

4.  Stairs.  Starts at "initial voltage," waits there for "time per

step" time, then ramps to next voltage in time "rise/fall time."

This occurs "number of steps" times, for a total of "steps"+1

voltages between "initial voltage" and "ending voltage,"

inclusive.  After the cycle, it ramps all the way back to

"initial voltage," again in "rise/fall time," and starts over.

If "initial delay" is nonzero, the waveform remains constant

for that long before beginning.

5.  Bistable switch.  If the switch is "off," output is "initial

voltage."  If it is "on," output is "pulsed voltage."  When the

switch is changed, output switches from initial to pulsed

voltage in time "rise time," or from pulsed to initial voltage

in time "fall time" (regardless of whether the voltage was

actually increasing or decreasing).  On RESET, the switch is

set to on or off according to "reset state."

6.  Monostable switch.  Output is normally "initial voltage."  When

switch is tapped, output ramps to "pulsed voltage" in time

"rise time," holds for "pulse width," them ramps back in time

"fall time."

THE FET7 MOS MODELS IN ANALOG

This release features a new family of MOS models, the FET7 series

(three-terminal gates PFET7T and NFET7T, and four-terminal gates

PFET7F and NFET7F) as well as several supporting gates. These

models are different from previous MOS models in analog, in structure as

well as form.

This Web page is a quick introduction to these models; for

complete details, see the paper log/lib/fet7model.ps or fet7model.pdf.

The model on which these gates are based is parameterized in a new

way. The parameters are properties of the physical construction of

devices. In addition, they are independent properties. Therefore, it makes

sense to vary one parameter while the others are left fixed. This is useful

in terms of finding values which give a good fit to a given process, and

also as a way of exploring the effect of process values on device and

circuit behavior.

The simulator has been modified so that the simulation environment itself

holds information about MOS physics and fabrication; in previous models,

each gate held this information redundantly. This information has been

broken up into several categories:

Temperature: The THERMAL gate lets the user view and

change the ambient chip temperature. Temperature-dependent

quantities in the MOS modeling system are automatically updated

to reflect temperature changes. Unfortunately, the ambient

temperature set by the THERMAL gate is not presently used by

gates outside the FET7 family, such as the DIODE1 gate.

Physical Constants: The PHYSICAL gate holds fundamental

physical quantities, such as the Boltzmann constant, and derived

quantities from these constants, such as kT/q. Derived quantities

are denoted by the annotation (Computed) in the parameter

description. These derived quantities cannot be directly changed

by the user, but will automatically be updated if the user changes

their underlying parameters.

Fabrication Parameters: The channel-specific fabrication

parameters for N- and P-channel devices, along with derived

quantities of interest, are displayed in the DEVTECHN and

DEVTECHP gates. Several channel-independent fabrication

parameters are in the PHYSICAL gate.

Run-Specific Parameters: The parameters in the DEVTECHN

and DEVTECHP gates are relatively constant from run to run of a

given process. In contrast, a small number of channel-specific

parameters do vary significantly from run to run, and from

transistor to transistor. The gate RUNSPEC provides a way to

change these parameters for all transistors in the simulation; in

addition, these parameters can be varied on a per-transistor

basis, to simulate various forms of device mismatch.

Transistor-Specific Parameters: The user can change the

geometry of each FET7 gate (drawn width and length in lambda,

source and drain area in square microns), and the offset

parameters mentioned above. Derived parameters (which may be

voltage and temperature dependent) are displayed for each

transistor (Vt, kappa, Early voltage and linear-lumped

capacitances). These parameters cannot be directly changed, but

reflect changes in their underlying parameters and voltages in real

time.

At start-up, the simulator reads in the default parameters for the FET7

series from the file log/lib/mos.cnf. The default mos.cnf file contains

parameter values suitable for the MOSIS SCN 1.2u process (SCN12, fab

run N52V); this file also appears as mosscn12.cnf. Parameter files for the

HP CMOS26G 0.8um process (mos26g.cnf) and the HP CMOS14TB

0.5um process (mos14tb.cnf) are also supplied.

The annotated file mos_example.cnf explains the process of converting

MOSIS-supplied documentation for a process to a cnf file. We hope

users will create cnf files for other processes, and send them along to us

for inclusion in later releases.

The LOGSPC utility for creating Spice files from Log schematics has

been updated to be FET7-aware. See the file log/lib/spctest.lgf for an

example of using FET7 transistors for SPICE-deck creation.

_______________________________________________________________________________

OTHER SIMULATION FEATURES

Simulation goodies

"Scope" mode

SIMULATION GOODIES

1.  If there are any circuit elements with unconnected pins, AnaLOG

will refuse to simulate.  If you meant to leave the pin

unconnected, you must connect it to something like a "dummy"

capacitor to ground.  If there are unconnected gates, the

NUMBERS icon will display a message of the form "OPAMP is

unconnected".  Digital LOG doesn't mind unconnected pins on

gates.

2.  If you give a ":DIM" command, AnaLOG gates will be drawn in dim

colors until they are fully connected.  Another ":DIM" turns this

mode off.  This command only works with the analog simulator.

3.  Select Probe in the Cursor menu (or press the "." key) to enable

Probe mode.  The cursor changes to a different style arrow; when

you move this arrow over a gate or wire, a message below the

AnaLOG NUMBERS icon shows the internal state of the gate and/or

node.  In Digital LOG, the lines at the bottom of the screen light

up according to the state of the node being probed.  Probe mode

also displays the name of a gate being touched.  Select again

to turn the mode off.  You can also press "e" or "x"

("Examine") for a temporary Probe mode which turns off

automatically when you click the right button.

4.  Select Glow in the Cursor menu (or press the "g" key) to enable

Glow mode.  Wires "glow" in colors according to the voltages

they carry.  In AnaLOG:

Black       means at (or below) ground.

Dark red    means within one transistor threshold of ground.

Red         means an intermediate voltage.

Pink        means within one transistor threshold of VDD.

White       means at (or above) VDD.

Green       means the wire is not driven by an analog gate.

In digital LOG:

Black       means the wire carries a Zero.

Red         means the wire carries a One.

Green       means the wire is not driven.

Glow mode may be left on at all times, though it may make the

simulation a little slower.

5.  Select Simulation in the Misc menu (or press the "o" key)

to turn the simulator off or back on again.  When simulation is

off, the gates and nodes of the circuit appear to freeze at their

last simulated values.  Turning the simulator back on resumes the

simulation.

6.  Press the "t" key to simulate for one time-step, then stop.

You can press t as many time as you wish to see the simulation

evolve, then press o to turn full-speed simulation back on.

7.  LOG tries to balance simulation speed against response time for

the mouse and keyboard.  If you let go of the mouse and don't press

any keys, LOG slowly shifts the balance to prefer simulation.

When you again use the mouse or keyboard the balance reverts to

favoring editing.  You can press the "f" key to switch

immediately to "fast" mode (preferring simulation over editing).

"SCOPE" MODE

1.  Numbers are great, but real people like to see plots of their

output waveforms.  In LOG, the mechanism for this is called

Scope mode.  To use it, first connect terminals (TO or FROM)

to each signal to be monitored and give the signals unique

names, then select SCOPE in the Misc menu, or press the "s"

key.  The scope can display both digital and analog signals.

2.  The Scope screen consists of a large field of dots representing

the "divisions" on a normal scope face, an area to the left of

the dots where signal names can go, and some menu words at the

bottom.  Use the mouse as usual to select menu words for the

functions that you want.  In particular, touch QUIT (there's

one on each side, for your convenience!) or hit <CNTRL>-C to

return to the circuit diagram.

3.  To display a certain signal on the scope, just type the name

of the signal and press ENTER.  The name will appear somewhere

at the left edge.  You can use the mouse to press and drag the

name to any vertical location.  To remove a signal name, drag

it off the top or bottom edge.

4.  Initially, the scope is not "triggered," so nothing is

displayed.  To begin taking data, touch Trigger.  The scope

also automatically triggers when you reset the simulation; there

is a handy Reset on the menu too.  A few moments after you

trigger the scope, lines to the right of the signal names

will begin to appear.

5.  In all probability, the lines will be absurdly wide or

ridiculously thin.  In other words, the time scale is wrong.

To fix this, press the "<" or ">" keys to shrink or expand

(respectively) the plot until it fits nicely on the screen.

You can also use the arrow keys to scroll around the plots in time;

by scrolling and expanding you can focus in on any part of the

simulation history.

6.  Touch Config to change the various Scope mode parameters.

You will find here the seconds-per-division and seconds-at-

left-edge parameters that the zoom and scroll commands

manipulate graphically; you can set them numerically if you

prefer.  The current time and timestep are displayed for

your reference in setting these values.

7.  Scope mode has a limit on the number of data points it will

take before stopping, initially 2000.  You can set this in

the Config screen, but beware that if you set it too high

AnaLOG will run out of memory and lose ALL of your data!.

This screen also displays the number of data points taken

so far.  What happens when this reaches the maximum depends

on the "trigger mode" attribute:

Manual:        Scope starts taking data when you touch

"Trigger," and stops when you touch "Trigger"

again, or when all the data points are used.

Beware that when memory fills, there is no way

to resume taking data except by retriggering

and losing all of your old data!  LOG prints

warning messages when you are about to run

out of data points.

On Reset:      Like Manual, but also triggers automatically

on RESET.  This is the default mode.

Triggered:     Like Manual, but also triggers automatically

when a triggering pulse is detected on the

signal that you specify.  Not relevant for

AnaLOG, because triggering on analog signals

has not yet been implemented.

Continuous:    Triggers manually or on RESET.  When data

memory is full, old data points are removed

from the left and new ones added to the

right.  Never stops triggering unless you

turn it off by hand.

On Reset mode is typically used when you are interested in the

first data points of a long simulation; Continuous mode is for

when you are interested in the most recent data points.

Note that the word "Trigger" is lit whenever the Scope is taking

data, and unlit otherwise.

8.  Normally, Scope mode records exactly one data point per timestep

returned by the simulator.  The "minimum timestep" and "maximum

timestep" attributes can modify this.  If you would like to

conserve memory and take data points only at, say, 100us intervals,

you could set "minimum timestep" to 100us.  Scope mode will then

record a data point only if it's been at least 100us since the last

point was taken.  The other attribute, "maximum timestep," does

not work with the analog simulator.

9.  When you touch (instead of dragging) a signal name on the left edge,

you Configure the parameters for that signal's particular trace.  In

AnaLOG, you can set volts-per-division, plus an "origin" voltage for

plotting small signals (for example, if you have a signal that

varies slightly around 2V, set the origin to 2V and the scale to,

say, 50mV per division).  You can also change the color of the

trace, very helpful for plotting several traces on top of one

another.  You can also set logarithmic mode, in which case you

specify "Vzero" which is the voltage to plot at the origin, with one

decimal order of magnitude per division above and below.

10.  To plot an AnaLOG current, you must take an ISCOPE gate, which is

the funny-looking meter with a hook coming out the back, connect it

to the pin to be measured, and connect a terminal (TO or FROM) to

the "hook." Now, when you monitor that name in Scope mode, you will

measure Amps instead of Volts.  Note that the "hook" pin of the

ISCOPE is a special "current-mode" data type, and any attempt to

connect it to a normal analog signal will fail.  If an ISCOPE ever

appears to undergo spontaneous combustion, it is because you put it

down with the hook touching an analog wire.

11.  If you try pressing down the mouse anywhere in the grid of dots,

you will get a horizontal and/or vertical line whose meaning

depends on two indicators at the bottom of the screen:

Absolute:      Display the value of time, or of the

currently-selected signal, that corresponds

to the mouse position.

Delta:         Display the difference in time or signal

value that corresponds to the distance between

the current mouse position, and the position at

which it was first pressed down.

Value:         Display the value of the current signal as

of the time that corresponds to the mouse

position.  For example, you move the mouse to

the "2.3ns" point, and the Scope displays the

value that the current signal had at that time.

Slope:         Display the slope of the line between the

starting and current mouse positions, where

rise is measured in the units of the current

signal, and run is measured in seconds.

Time        Display absolute or delta time, in seconds.

Freq        Display delta time, in Hz (= 1/seconds).

Signal      Display the value of the signal whose name is

highlighted, in Volts or Amps.  As you touch

this word, each signal is highlighted one by

one.  You can also select a current signal

by tapping its trace on the screen.

12.  The word ON is a "power switch" for the simulation.  If you touch

it, it changes to OFF and the simulation halts.  When you touch it

again, simulation continues.  (In the circuit-diagram screen, this

function is called Simulation in the Misc menu.)

13.  When you touch Dump, the Scope writes out all of the data into a

file in a standard format.  You may want to do this in order to

use a more hefty data analysis tool with data generated by AnaLOG.

The CNS 182 "View" program is one such tool, with a special "aload"

command for loading AnaLOG dump files.

_______________________________________________________________________________

PLOTTING

Plotting a circuit

Plotting your data

Customizing your plots

PLOTTING A CIRCUIT

1.  To make a hardcopy plot of your circuit, press "p" or select

Plotting in the Misc menu.  You get a screen with an (admittedly

ugly) view of what your circuit will look like on paper.

2.  To zoom in on an area, just use the mouse to sweep out a rectangle

around that area.  Press Zoom out to return to viewing the

full circuit.  The Zoom feature uses "markers," which are little

red brackets that show up in the circuit diagram.  You can move

the markers around in circuit-editing mode, too, and turn them

on or off with the Markers command in Frills.

3.  Touch Config to configure the plotter.  You can change the

font that is used for labels (there is a wide selection of fonts

available, though most of them are silly).  You can select how

the plot will be oriented on paper (default is to let LOG choose

the orientation to make it come out as large as possible).  You

can set the size of the paper if your plotter is too primitive

to know that for itself.  (For the laser printer, the printer

always determines the paper size itself.)

4.  Touch Options to configure the circuit-diagram plot.  Here you can

adjust the text sizes, border and box styles, solder dot size and

line width.  The Gate text limit specifies the minimum size of gate

text attributes to display on the screen.  Gates have a default text

size of 50, set in loged, so by default they are not displayed.

5.  When you are all ready press File and away it goes, to the system

Postscript printer. The PLOT button used to control pen plotters

directly, but now a better way of using pen plotters involves

plotting into a file (see 6 below) using HPGL.  [NOTE: The Unix

version is not able to drive pen-plotters directly.]

6.  You can also plot into a PostScript file, for inclusion, say, in

a TeX document.  Set the file name in Config, then touch File.

PLOTTING YOUR DATA

1.  Take data on all interesting signals using the Scope screen.

2.  To get a high-quality plot, touch the word PLOT on the SCOPE screen.

This switches to an alternate menu of commands.

3.  The word "Y-axis" is lit up, meaning that if you touch any signal

name or signal trace, or the word "Time," or type any signal name

or arithmetic expression involving signals, then that signal will

go on the Y axis of the graph.

4.  When "X-axis" is lit, you are selecting what should go on the

X axis of the graph.  After both axes have been specified, you can

change your mind by touching one of the two words, then touching a

new signal.

5.  Touch Plot to run the plotting program.

6.  In the plotting program, touch Config for general plotting options,

or Options for options specific to data plotting.

7.  In the Options screen, you get function name, plus label, units,

log/linear, and minimum and maximum values for each axis.  If you

don't specify a min and max, they are calculated from the data.

You can enter any arithmetic expression as the function; variables

in the expression may be any signal name from the Scope screen, or

Time, or one of A through F.  These last are user-defined parameters

you can change in the Variables screen.

8.  You can also specify line style, and a symbol to plot on data points.

9.  Normally the axis labels are drawn at a larger size so that you can

read the numbers and exponents.  When plotting on paper, the axis

labels are regular-size.

10.  Zoom out just clears the min and max for both axes to blank.

11.  Touch Plot to send the current graph to a local plotter.

12.  Touch File to send the current graph to a file or device, as

described in the Config screen.  Default is the Apple LaserWriter.

13.  The simulation is still running while you are in the plotting tool.

If the scope is still triggered, the set of data plotted will be

those taken as of the time the screen was last refreshed, or the

Plot button was pushed.  Thus if you just sit and press the space

bar, the plot will be slightly different each time.  If this is

confusing, we suggest turning the simulator off first by changing

ON to OFF in the Scope menu.

14.  Touch QUIT to return to the Scope.

15.  For more advanced data munching, touch instead "Dump" from the SCOPE

menu, then give some file name, by pressing the "!" key) and run the

"View" program also written at Caltech.  This program has a "load"

command which can read the Dump file, using signal names as curves.

(Note that the Dump command appends to the dump file if it already

exists.  "Load" will take the most recent curve if the file contains

several by the same name.)

CUSTOMIZING YOUR PLOT

1.  When LOG starts up, it reads lplot.cnf in the /lib directory (ask

your local anaLOG maintainer), which contains certain commands for

setting up the plotting system.  This file in turn reads pens.cnf,

which has mappings of colors onto pen numbers.  Each of these

commands begins with "lplot:", which signifies that the command is

intended for the LOG plotting tool.

2.  You can give these commands yourself, either while LOG is running

by pressing ":" and then typing, e.g., "lplot: size B", or by

including such a line in your own "log.cnf" file.

3.  The most interesting configurable thing for the plotting tool

is color.  LPLOT uses a two-step method for determining color.

Each kind of object has a particular "color-name".  These

color names (like GATE or WIRE) are looked up in the color-name

table to produce a pen-name (like RED or BLACK), which in turn

is looked up in the pen-name table to get the actual pen

number to use for pen plotters.

4.  The following commands may be used after the word "lplot:".

LPLOT: BORDERSTYLE style

Specify the line style used to draw the border.

Default is 1.

LPLOT: BOXSTYLE style

Specify the line style used to draw the boxes.

Default is 2.

LPLOT: COLOR  color-name  pen-name

This maps a given color-name onto a given pen-name.

Standard pen names are "black", "red", "green",

"purple", "blue", "yellow", and "none" (which

suppresses drawing of that color).  You can also

specify a second color-name in place of a pen-name,

in which case the first color-name uses the same

pen as the one which was recorded for the second.

LPLOT: COLORS

Display a list of all color names that have been

recorded in "LPLOT: COLOR" commands.

LPLOT: CONFIG

Open up the plotter configuration attribute screen.

This is the same as touching "Config" while in the

plot mode.

LPLOT: DOTS  gate-name  gate-name  ...

Undo the effect of previous "NODOTS" commands for

the specified gates.

LPLOT: FILE psfilename

Specify the filename for saving or printing a PostScript

plot.  The ".ps" is automatically postpended.

LPLOT: FONT  font-file

Use the specified font file for text.  The default

file is "lplot.font" in the /lib directory for log.

LPLOT: GATEFONTSIZE size

Specify the size of the gate font.  Default is 12.

LPLOT: GATETEXTLIMIT size

Specify the minimum size of gate text attributes to

display on the screen.  Default is 60.

LPLOT: INVISIBLE  gate-name  gate-name  ...

Add the specified gates to the list of gates which

do not show up in plots.  By default, TIME, NUMBERS,

and DIGH are on this list.

LPLOT: LABELFONT  font-number

Specify the font number to use for text labels in

plots.  For the standard font file, this should be

a number from 1 to 13.  Common numbers are 1 for a

very simple font, 2 for a standard Roman font, and

3 for Roman italics.  Default is 2.

LPLOT: LABELFONTSIZE size

Specify the size of label text.  Default is 16.

LPLOT: LINEWIDTH width

Specify the width of lines drawn by a Postscript plot.

Default is 7.

LPLOT: NODEFONTSIZE size

Specify the size of node number text.  Default is 8.

LPLOT: NODOTS  gate-name  gate-name  ...

Add the specified gates to the list of gates for

which extra solder blobs will not be generated.

By default, only the digital LED gate is on this

list.

LPLOT: PENCOLOR  pen-name  pen-number

This specifies the mapping of pen names (such as RED or

BLACK) onto pen numbers for the plotter.  The standard

configuration file "pens.cnf", which is included by all

the standard "log.cnf" files, contains good default

PENCOLOR commands.

LPLOT: PENS

Display a list of all pen names that have been

recorded in "LPLOT: PENCOLOR" commands.

LPLOT: PLOTTER  hpib-address

When using a pen plotter on the HPIB bus, this

command specifies the HPIB address of the plotter.

Default is 5.

LPLOT: PSFILE psfilename

Specify the filename or command for saving or

printing a PostScript plot.  This can either be

a regular filename, such as "plot.ps", or a

filename piped to a command, such as "/tmp/*.ps | lpr *"

(the default).

LPLOT: SIGFONT  font-number

Specify the font number to use for signal names

in plots.  (See LABELFONT command.)  Default is 2.

LPLOT: SIGNALFONTSIZE size

Specify the size of signal text.  Default is 18.

LPLOT: SIZE  paper-size

Set the size for paper in pen plotters.  Other output

devices, such as the laser printer, compute the size

on their own.  Size A is standard 8.5" by 11" sheets;

size B is about 11" by 17".  Size AUTO means to

compute the size automatically.

LPLOT: VISIBLE  gate-name  gate-name  ...

Remove the specified names from the list of

INVISIBLE gates.

5.  Here is a list of gate-color names used by LPLOT.  Where an

alternate name is given, LPLOT looks up that color-name if

the main color-name is not found.  The ultimate default is

pen-name BLACK.  Except for SCOPE and BORDER, all of these

apply only to circuit-diagram plots.

BORDER     Color of the dotted border on the screen that indicates

the limits of the page.  Default in "lplot.cnf"

is RED.

DASHBOX    (Alternate: LABELTEXT.)  Color of "yellow boxes" (as

gotten from the Frills menu).

GATE       Color of regular gates.

LABELTEXT  (Alternate: DASHBOX.)  Color of labels (as gotten

from the Frills menu).

NODE       (Alternate: GATE.)  Color of node names, drawn when

requested by the Options screen for gate plotting.

Node names are invented by LPLOT and do not

correspond to node names used by any other program.

OFFLED     (Alternate: GATE.)  Color of digital LED's and other

readouts which are "off".  Default is YELLOW.

ONLED      (Alternate: GATE.)  Color of digital LED's and other

readouts which are "on".  Default is RED.

SCOPE      Color of data plots.

SIGNAL     (Alternate: GATE.)  Color of signal names in TO and FROM.

SOLDER     (Alternate: WIRE.)  Color of solder blobs.

WIRE       Color of wires.

_______________________________________________________________________________

LOG-TO-NTK CONVERSION

Overview

LOGNTK commands

Identifying things

LOGNTK messages

Notes

OVERVIEW OF LOGNTK

1.  LOGNTK is a program to convert diagrams in AnaLOG (.LGF files)

into network descriptions of circuits (.NTK files).  When you give

the ":LOGNTK" command in AnaLOG, it reads the diagram on the

screen, treating the AnaLOG or Digital LOG transistors as NTK

transistors, other Analog gates as NTK cells, and certain labels as

LOGNTK commands.  Only labels surrounded by < and > or in quotation

marks are significant to LOGNTK.

2.  LOGNTK manages hierarchical definitions.  The notation used for

hierarchy is the same as that used by the Digital LOG simulator.

LOGNTK sees the world as cells; each cell turns into one NTK

file.  To define a cell, draw a circuit diagram, add a label which

consists of the cell name enclosed in quotes (as in "Fred"), then

define the cell's connectivity either with a template gate or with

a port list label.  To put several definitions on a page, enclose

each definition in a dashed box (from the Frills menu).  All of this

is described in the documentation for the digital simulator.

3.  One difference is that while in Digital LOG you normally use INST

gates (like INST3), in LOGNTK you normally use "generic instance"

GINST gates (like GINST3) instead.  LOGNTK recognizes both families

of instance gates, but the INST gates can connect only to digital

components, whereas the GINST gates can connect to anything.

4.  A complete project in LOGNTK may include any number of cells.  The

top-level cell describes the whole circuit and includes instances

of the other cells, which may themselves contain instances, etc.

Anything goes, as long as no circular references exist among the

instances.  At the bottom of the hierarchy are transistors and/or

primitive gates.  Examples of transistors are the AnaLOG gates

NFET5 and PFET5, and the Digital LOG gates V_NFET and V_PFET.

Examples of primitive gates are all the A_ gates from the ACTEL

gate library.

5.  Each NTK file produced by LOGNTK is completely self-contained.

When you LOGNTK a cell that has instances of another cell, the

program copies the definition of that cell into the output file.

An NTK file consists of any number of cell definitions, followed

by a "main program" which calls the outermost cells.  If your

top-level LOGNTK page does not include a cell name, that page

defines the "main program" for the project.  If you do name your

top-level page, LOGNTK puts that page in a cell as usual, and

then creates a dummy main program which "calls" that cell once.

Most simple programs like NETCMP, the NTK file comparator, do

not care about this distinction.

6.  When you give the ":LOGNTK" command, LOG converts the definition

on the current page into an NTK file.  The file name is usually

derived from the cell name, with ".ntk" appended.  (The name of

the LGF file that contains the circuit diagram is irrelevant.)

If there are several cell definitions on the page, LOGNTK does

each of them in turn.  You can also pick a specific definition

by typing ":LOGNTK cell-name", where "cell-name" is one of the

definitions on the current page.  (You can give a list of cell

names, and the names may include "*" and "?" wildcards.)

LOGNTK COMMANDS

1.  LOGNTK diagrams often contain additional commands in the form of

labels.  These labels are always surrounded by "<" and ">" brackets.

Any labels of this form, but with commands that LOGNTK cannot

recognize, are simply ignored.  LOGNTK assumes these commands belong

to some other program, such as Digital LOG's hierarchy compiler.

2.  You can also include these commands in your "log.cnf" file,

preceded by "logntk:".  For example, the label <global clock*>

defines global signals just for one cell; the CNF command

"logntk: global clock*" defines global signals permanently.

Finally, you could also press the ":" and type the

"logntk: global clock*" command during your LOG session.

3.  Certain arguments may include wildcard characters.  LOGNTK uses the

same kinds of wildcards as Unix: "?" matches any character, "*"

matches any zero or more characters, and "[ABC]" matches either

A, B, or C.  Also, most names in LOGNTK are case-insensitive, but

LOGNTK still strives to duplicate the upper/lower-casing that you

use into the output file, and always capitalizes all the uses of a

name consistently.

4.  Every LOGNTK definition needs at least some commands.  To create a

named cell, you need a NAME command (or a quoted label; see below),

and either a PORT command or a template gate.  For an unnamed

top-level cell, you need at least a FILE command.

5.  Here is the list of LOGNTK commands.  The ":" following the command

names are optional.

GLOBAL: signal-names

This command lists names which should be "global" to

the entire project.  For example, VDD and GND are made

global by the standard "logntk.cnf" file.

This means that if you create a cell which includes

VDD and GND gates, those nodes will automatically be

made into "hidden" ports on the cell.  Instances of

that cell will gate VDD and GND wired to them

automatically.  All you have to do is used VDD and GND

naturally, and let the system keep track of it for you.

You can declare your own TO/FROM signal names to be

global in the same way.  The "signal-names" are case-

insensitive, and may include wildcard characters.

PORT: signal-names

This command gives the list of ports in the cell.  It

may be used in place of a "template" or "example" gate.

Port lists are supported for historical compatibility,

but future designs should probably use template gates

instead.

Each name in the list should correspond to a signal name

in the circuit diagram.  VDD and GND refer to the VDD and

GND circuit symbols.  The position of a name in the port

list, not counting global names, corresponds to the pin

number of the associated instance gate.  For example, if

the .GATE picture for an inverter has the input as pin 1

and the output as pin 2, then the appropriate command for

the inverter's circuit diagram would be:

<port: in out vdd gnd>     or

<port: gnd vdd in out>     or whatever.

The positioning and ordering of global names in the list

is irrelevant, but any global signal that is used by the

cell or by its sub-cells must be present.  (When you use

a template gate instead of a port list, LOGNTK manages

all this for you.)  If you give a ":LOGNTK" command and

their is neither a port label nor a template gate in the

definition, LOGNTK will sort the signal names into

alphabetic order and create a PORT label.  You will

probably need to edit this command to put the ports in

the proper order.

If several PORT labels appear in the cell definition,

they are appended together in an undefined order.  If

you don't mind this uncertainty, LOGNTK can handle an

unlimited number of ports.

ORPHAN: gate-names

This command gives the gate(s) which are used to

represent "orphan" nodes, i.e., nodes that are not

connected to any transistors or subcells.  By default,

the built-in gate CIRC1 is considered an orphan gate, so

that a circuit with five CIRC1's will have five orphan

nodes in its NTK file.  The gate-names may use wildcards.

IGNORE: gate-names

This command gives a list of gates to be ignored.  This

is rarely necessary, because if LOGNTK sees a gate which

has not been defined as a transistor or cell, and for

which no "gatename.NTK" file exists, then it will

automatically ignore that gate with a warning message.

This command suppresses the warning.  The gate-names

may include wildcards.

NAME: cell-name

This specifies the name of the cell in the NTK file.

This is an alternate form for the quoted "cell-name"

label.  There must be exactly one such label for each

cell.

FILE: file-name

This specifies the name of the output file.  The suffix

".NTK" will be added if necessary.  If you omit the FILE

command, the file name will be taken from the cell name.

(Every LOGNTK page must specify either a cell name or

a file name, or both.)

CELL: gate-name  cell-name  file-name

This command links gates in the circuit diagram

to the NTK files they represent.  A good idea is

to use the same name for the gate, cell, and file;

if you give only one name in the command, that

name will be used for all three.  One time you

might need to give the long form is if the NTK

file is not in the standard directories, so you

need to give a full path for it:

<cell: opamp tc_amp /lib/wollib/stdlib/opamp>

In this example, OPAMP gates, or generic instance

gates programmed as OPAMP's, will convert to calls

to the cell TC_AMP, which may be found in the .NTK

file specified.  CELL commands describing cells that

are never instantiated in a file are ignored.

LIB: directories

This command names directories in which .NTK files

for sub-cells may be found.  LOGNTK looks first in

the current directory, then in each of these

directories in turn.

SIZE: number

This specifies the sizes of nodes.  Currently,

there is no way to override this for particular

nodes.  The default size of 1 should be sufficient

for CMOS users.  Most NTK-reading programs ignore

node sizes, anyway.

STRENGTH: number

This specifies the strengths for transistors.

This can be overridden as described for TRANS.

The default strength is 2.  Most NTK-reading

programs ignore transistor strengths.

TRANS: gate-name  type  strength

This command declares transistor symbols.  If you

need to make your own transistor gates, you will

need to use this command.  The "type" is one of

N, P, or D.  The strength is optional; if it is

a positive number N, then the strength of the

transistor is N; if it is missing, the strength

is the default value; if it is -N, the strength

is found in attribute number N of the transistor

gate, which must be an integer-valued attribute.

The file logntk.cnf contains TRANS commands

for all the standard LOG transistors.

PRIMITIVE: gate-name  cell-name

This command declares primitive gates, gates which

should be converted into instances of cells which

have no definition.  If the gate has N pins, then

the output file will call the cell with N ports,

in order of pin numbers on the gate.  For example,

<primitive: a_and2 and2>

Causes the ACTEL gate A_AND2 to convert to a call

to cell AND2.  Both names may contain wildcards;

the actual commands in logntk.cnf are:

logntk:  primitive  a_*  *

logntk:  primitive  a_tribuf  tribuff

which maps A_AND2 to AND2, A_DFC to DFC, etc.,

and A_TRIBUF to TRIBUFF (a special case).  Note

that more specific names come after more general

names.  The command PRIMITIVE may be abbreviated

as PRIM.

TOP        Force this to be a top-level NTK file, that is,

after the main cell has been defined, one "call"

to the cell is also written into the file.  The

NETCMP program can only read top-level NTK files.

This mode is the default; you only need the TOP

command to override an earlier LEAF command.

LEAF       Force this to be a leaf-level NTK file, that is,

only the cell's definition is included (plus

definitions for any subcells used).  There is

usually no reason to use LEAF, since most programs

can read top-level NTK files and ignore the top part.

NOTOP      Set a compatibility mode in which a LOGNTK command

on a page with no NAME command (or quoted name label)

will produce an error instead of creating a top-

level NTK file.  If you are a stick-in-the-mud like

John Lazzaro, you can put this in your log.cnf file.

IDENTIFYING THINGS

1.  Cells and nodes in the NTK file are assigned names automatically by

LOGNTK.  The one exception is that if a node has a signal name

associated with it, that name is used.  The :IDENTIFY command in

AnaLOG lets you find out what names correspond to what circuit

objects.  Type:

:IDENTIFY

and move the cursor over any node or gate.  The associated name

appears at the bottom of the screen.  The IDENTIFY command is

finished when you press the mouse left button down, or hit the

<CNTRL>-C or SELECT key.

2.  NOTE:  If you change anything in the circuit between :LOGNTK and

:IDENTIFY, the names chosen may not be the same for the two

commands.  This is true even if you changed it and then put it

back the way it was.

3.  You can also type:

:IDENTIFY name name name ...

to "light up" the gates or nodes corresponding to those names.

You can press the space bar to refresh the screen afterwards.

LOGNTK MESSAGES

1.  LOGNTK displays the following messages at the top of the screen

during operation.  If there is a "fatal" message, the output NTK

file will not be complete.

Can't understand command: FOO

This command appeared in a LOGNTK: line in the LOG.CNF

file, and it is not one of the command words listed

above.  Unrecognized commands in < > labels are

ignored, since it is assumed that they belong to some

other LOGNTK-like program.

Syntax error in TRANS command

The transistor-type argument is missing or wrong.

Cell file FOO.NTK not found     (fatal, unless next message

is shown too)

The file named in a CELL command does not exist.

Using file FOO.NTK instead of file BAR.NTK

The CELL command listed a file which does not exist,

but the "gatename.NTK" algorithm found the file.  You

should edit the label accordingly.  (The above example

corresponds to a command of <cell:  foo xxx bar>.)

<cell: FOO>

This is a label added if the "gatename.NTK" algorithm

worked and there was no CELL command for this gate.

This just serves as a reminder that LOGNTK made this

assumption for you.

No cell or file name!                             (fatal)

<file: (place file name here)>

There were no <name: xxx>, <file: xxx> or "xxx" labels

on the page.  LOGNTK assumes you are making a top-level

file, and creates a <file> label which you can edit.

If you are making a cell instead, throw away the <file>

label and make a cell-name label.

No cell name!                                     (fatal)

<name: (place cell name here)>

No name was specified for this cell.  You are given a

label which you can edit to specify the cell name.

This error occurs only in NOTOP mode.

More than one <name: xxx> or "xxx" label on the page!     (fatal)

Multiple cell-name labels appeared, not enclosed in

dashed boxes.

Cell FOO is recursively defined!                  (fatal)

LOGNTK detects some, but not all, such errors.

Warning: Port FOO does not appear in the circuit

A name in the PORT command does not correspond to any

signal name in the circuit.  This may be because your

subcells use GND and VDD but you don't explicitly use

them here (not an error), or it may be because you

forgot to label part of your circuit.

Signal FOO is missing from the port list          (fatal)

Global node FOO is not a port

You used a cell that requires a certain global signal,

such as VDD or GND, which you omitted from your own

PORT list.  All global signals used by a cell must be

passed down through all higher-level cells in the

hierarchy.

Warning: FOO appears more than once on port list

You included the same name more than once in a PORT

command.  The NTK file format has no way to express

that two ports of a cell are shorted together, so

LOGNTK can not write an NTK file for such a cell.

The later occurrence of the name on the port list

is ignored.

Template has pins shorted together                (fatal)

Analogous problem as "FOO appears more than once

on port list", but for template-defined gates.

Warning: Template gate has no connections

This is not strictly an error, but in NTK a cell

with no ports is usually a sign of trouble.

Using port list: A B C D

If the cell was defined with a template gate, this

message shows the order in which the signals were

assigned as ports, just for your information.

Port list ignored for top-level NTK file

This is a top-level file (not a cell), but there

was a PORT command present.  The PORT command is

ignored.

More ports than pins in cell FOO                  (fatal)

Too few connections in cell FOO                   (fatal)

The NTK file for subcell FOO listed more non-global

ports than there are pins (on an instance gate

created in LOGED) or connections (on a generic

instance gate).  This could be because the .GATE

and .LGF files for that subcell are out of sync, or

because you forgot to list some of the ports as

GLOBAL.

More pins than ports in cell FOO                  (fatal)

Too many connections in cell FOO

The NTK file for subcell FOO listed too few non-global

ports.

Warning: Gate FOO was ignored

A gate of type FOO was ignored because no definition

for it could be found, but that gate was not listed

in an IGNORE command.

No nodes or gates called FOO

An :IDENTIFY command gave a name which does not exist.

2.  There are also a few "informational" messages which can be turned

off by giving a ":VERBOSE OFF" command in LOG.

NOTES

1.  LOG nodes which are not connected to any transistors or cells

are ignored.  Thus, you can not get orphan nodes just by drawing

stray wires.  You must explicitly use "orphan" gates as described

above under ORPHAN.

2.  Cells which pass "global" nodes to their subcells, but which do

not actually use those nodes themselves, must include those

global names in their own port lists.  For example, if a

higher-level cell does not use GND and VDD but its subcells do,

you must include GND and VDD in this cell's PORT list.  (Note

that if you are using the recommended "template gate" method

rather than writing a PORT list, this is all handled for you

automatically.)

3.  LOGNTK is case-insensitive, but case-preserving.  That is, if you

call something FOO in some places, and Foo in others, the names

will be considered the same, and one of the names will be chosen

for use everywhere in the output file.  Which name is chosen is

arbitrary, except that preference is given to usage in the PORT

command.  Note that LOG itself is case-sensitive in some ways,

for example, TO/FROM signal names "Vdd" and "VDD" are considered

to be different by LOG but this difference is not acknowledged by

LOGNTK.  This sort of disagreement can cause LOGNTK to produce

strange or erroneous output files.

4.  The "VDD" and "GND" symbols are exactly equivalent to TO's or

FROM's with the names Vdd and Gnd, respectively.

5.  In LOGED, you can use the CONNECT command to cause two pins to be

connected internally.  (For example, the crossing-wires gate

CROSS2 and the toggle switch SW2 work this way.) If you use such

a gate as an instance in LOGNTK, only the lowest-numbered of the

connected pins corresponds to a port in the PORT list; the other

pins in the connection group are skipped when being matched to

ports, in the same way as global ports are skipped when being

matched to pins.  Once again, using a template gate to describe

your ports handles this automatically.

_______________________________________________________________________________

LOG-TO-SPICE CONVERSION

Description

Logspc Additions

DESCRIPTION

Harold Levy of Caltech modified the LOGNTK program to create a

new program, LOGSPC, that creates SPICE decks instead of .ntk files.

The tool is used identically to LOGNTK, so the manual section above

describes LOGSPC also. The command to invoke the tool is :logspc, and

configuration information is in the file lib/logspc.cnf. The FET

models NSPC1 and PSPC1, contributed by Bhusan Gupta at Caltech, are

perfect for use with LOGSPC.  These gates also simulate in analog;

however, these models are considered "experimental" as opposed to

"standard" models in analog, and are not supported by the chipmunk

maintainers.

LOGSPC ADDITIONS

A few additional commands have been built into LOGSPC to better

support SPICE capabilities.  Note that the SIZE and STRENGTH commands

of LOGNTK are ignored in LOGSPC, and that the following commands are

used instead:

NBULK: signal-name

PBULK: signal-name

These commands assign explicit names to the implicit

bulk terminals for 3-terminal FET (e.g. p/n-FETs 4 and 5)

and bipolar (e.g. NPN1, PNP1) models.  The signal-name

is output in capitals unless quoted.

LAMBDA: number

This command assigns an explicit lambda value (in

microns) to the FET7 model series.

CHLEN: number

This command assigns an explicit channel length (in

microns) to FET models using w/l ratios (e.g. p/n-FETs

4, 5, and 6).

PRE: filename

POST: filename

Specifies a file for inclusion into .spc files before

and after netlist.

Also note that as of LOGSPC version 0.5b the ability to extract VDIFF

gates is provided.  This feature may be shut off with the command

LOGSPC: IGNORE VDIFF

Finally, the TRANS command has been extended, to supporting adding new

gate types to logspc, as shown in the example below.

EXAMPLE

Copy the files chipmunk/log/lib/{spctest.lgf,pre.spc,post.spc} to your

directory, and examine and run logspc on spctest.lgf, to see the features

described above in action.

FILE CONVERSIONS

The supplementary toolkit includes companion programs to LOGSCP,

that simplify using LOGSPC output with other tools. These tools are located

in the spc-tools subdirectory of the toolkit. Three scripts are included:

sf:  Flattens hierarchical LOGSPC (version 1.0) output.

ss:  Converts output from "sf" into UCB SIM format for use with

UW's "gemini" layout-versus-schematic (LVS) comparison program.

ucbsim:  Appends "format: UCB" to the first line of SIM files generated

by ext2sim.

_______________________________________________________________________________

THE LOGED PROGRAM

Introduction

List of commands

DRAW command

LABEL command

DEF command

Digital gate definition language

Text file format

.GATE file format

INTRODUCTION

1.  The LOGED program is responsible for maintaining the libraries of

gates stored in .GATE files.  When LOG starts, the GATES commands

in its configuration file specify which gate libraries to use.

2.  The "loged" command may be given by itself or with a the name of

a file to load initially.  Additional arguments are interpreted

as they would be for a LOAD command, as a list of gates to load

from the file.  Note that in LOGED terms, a "gate" is a kind of

gate, such as NAND or 74138, not an instance of the gate-kind.

To facilitate scripted LOGED operations, a -p switch may be

provided as the first argument to LOGED.  This directs the program

to read commands from the standard input instead of the newcrt window.

3.  To exit LOGED, type QUIT.  You will be given a chance to save the

gates in memory if they have been modified.  If the gates came from

one file originally, LOGED uses that name.  Otherwise, if no file

was loaded or several files were mixed together, LOGED prompts for

the file name to use.

4.  At any time in LOGED there is a "current gate."  Many commands

operate on the current gate.  The GATE, NEXT or PREV commands can

be used to select the current gate.  LOGED stores the following

information for each gate:

a. The name of the gate, up to eight characters (uppercase).

b. The group number (library-screen page number), from 0 to 8.

c. The gate's picture, including pins and grab box.

d. The "simtype" number, telling which simulator "owns" the gate.

e. The gate's definition or program, simulator-dependent.

f. Connectivity information in case pins are hard-wired together.

g. Miscellaneous flags affecting LOG's treatment of the gate.

h. The gate's labels screen, shown when configuring the gate.

Various commands described below manipulate all these parameters

for the current gate.

5.  Many commands accept gate-name arguments which may include

wildcard characters.  The '.' and '*' characters stand for exactly

one, or zero or more, characters respectively.  Use '%' or '?'

(respectively) instead for wildcards which prompt yes-or-no for

each potential match.  A terminating ';' followed by digits

matches only gates in the groups specified.  For example, the

pattern 'a*bc;23' matches only gates whose names begin with 'a'

and end with 'bc', and which are in groups 2 or 3.  If a list of

several names is given separated by spaces, gates which match

any of those names are accepted.

LIST OF COMMANDS

1.  A complete list of LOGED top-level commands follows.  Some

commands are discussed in detail in later sections.

2.  Command and gate names are case-insensitive.  Commands can

usually be abbreviated to one or two letters.

3.  In addition to these commands, you can execute any Shell

command by preceding it with a '!' character, or enter an

interactive sub-shell by typing '!' alone.  Type QUIT to

the sub-shell to return to LOGED.

CONNECT  num num num ...

Connect the specified pins together in the current

gate.  When an instance of the gate is made in LOG,

these pins will be forced to the same electrical node.

For example, the CROSS gates in the standard library

emulate crossing wires by having pairs of connected

pins.  The pins of a gate are divided into connectivity

classes; initially every pin is in its own class.  The

CONNECT command with two or more numbers joins the

classes of the specified pins.  CONNECT with one pin

number unconnects the pin, splitting it back into its

own class.  CONNECT with no arguments displays the

current connectivity of the gate.

COPY  new-name

Make a copy of the current gate under the specified

name.  All aspects of that gate are identical to the

current gate, except the name.  The new copy becomes

the new current gate.

DEF        Edit the simulator's definition for this gate.  Exact

effect depends upon the current simtype of the gate.

This command only works of the relevant simulator is

available.  The type 0, 1, and 7 DEF commands always

work; type 16 and 32 commands work only if DIGLOG or

ANALOG have been permed, respectively.  (Note that

ANALOG actually supports both simulators.)

DELETE     Delete the current gate.  Given a name, deletes all

gates which match the name.

DO  name  command-line

For each gate that matches the specified name, make

it the current gate and execute the LOGED command

shown.  This command must not delete or rename the

gate; to delete multiple gates, use DELETE directly.

DRAW       Edit the current gate's picture.  Defined below.

DUMP       Print a "data sheet" for the current gate on the laser

printer.  If a file name is given, the PostScript for

the data sheet is written to the file.

EDIT       Edit the current gate's information using the text

editor, CAGED.  Given a name, edits all gates which

match the name.  The information is written into a

temporary text file as if by the WRITE command;

CAGED is run on the file; when the editor returns,

the file is reloaded as if by a READ command.

FLAG  flag-name

Turn the specified flag on or off for the current gate.

Flag names are case-insensitive and may be abbreviated.

NOFLIP   This disables flipping or rotation of

the gate.  Tapping on the gate always

acts as if CNFG mode were selected.

TOGGLE   The gate is a toggle switch.  When an

instance is tapped, it always mirrors

about its (original) y-axis.  The gate

can still be rotated in the menu area.

VISIBLE  The gate is visible even in Invisible

gates mode.

NAMED    The gate has an editable signal name,

like TO or FROM.  Should be used only

for simtype 1 gates.

NRIGHT   The gate's name is on the right, as in

the TO gate.  Used with NAMED, above.

NOSOLDER When plotted, the gate should not have

solder blobs added for T-connections

involving the pins.

USER1    A user-definable flag.  A simulator, tool,

or individual gate program can interpret

this flag in any way desired.  Currently,

this flag is unused.

USER2    Another user-definable flag.

With no argument, this command just displays the current

flag settings.

GATE       Switch to a new current gate.  If the specified gate does

not exist, a new "blank" gate is created.  (Often it is

easier to create a new gate by copying an existing,

similar one.)  The NEXT command can be used to switch to

a new gate with no danger of accidentally creating one.

GROUP n    Set the current gate's group number to "n", an integer

from 0 to 8.  With no argument, displays the current

group number.  The group number determines which

library page the gate appears in.  Group numbers for

the standard libraries are:

0        General-purpose gates

1        Generic digital gates

2        7400's series digital gates

3        (unassigned)

4        Analog gates

5        Digital VLSI gates

6        Digital ACTEL gates

7        (unassigned)

8        (obsolete gates)

HELP       Run the Chipmunk Help System on this help file.

LABEL      Edit the current gate's labels and attributes.  See

below.

LIB        Display a list of all gates, or the gates which match

the specified names.

LOAD  file-name  gate-name gate-name ...

Load the gates in a file into memory.  If gate-names are

specified, only those gates which match the names are

loaded.  By default a ".gate" extension is added to the

name; to load from a textual file, specify ".text"

explicitly.

MEMORY     Display the amount of memory free.

NEXT       Switch to the next gate in alphabetical order, or to the

next gate which matches the specified name.

PREV       Switch to the previous gate in alphabetical order, or to

the previous gate that matches the specified name.

QUIT       (or EXIT.)  Leave the LOGED program.  If any gates have

been changed, you are given the option to save them first.

READ       Much the same as LOAD, except the default file name

extension is ".def", typically a file written by the

"Write" option of the digital hierarchy compiler.

RENAME  new-name

Change the name of the current gate to the new name.

If a gate by that name already exists in memory, you

are given the option to replace it.

SAVE  file-name  gate-name gate-name ...

Save all gates in memory, or only the specified gates,

into a file.  With no arguments, saves all gates back

into the original file.  A ".gate" extension is appended

by default.  If the extension is not ".gate", the file

is written in a textual format readable by humans.

SIMTYPE n  Set the simulator number of the gate to "n".  This is

an integer from 0 to 255 which identifies the simulator

responsible for the gate.  Currently assigned simtypes

include:

0        General-purpose, "inert" gates.  Used

labeling (e.g., ARROW) and connection

(e.g., CRUNCH) gates.

1        Signal-name gates.  Pin 1 of the gate

is connected to a certain signal name,

which is either hard-wired (as in VDD

and GND) or configurable (as in TO

and FROM).

7        Self-sufficient Pascal gates.  The

code which defines this gate is given

complete access to LOG's simulator

interface.  For example, the TIME gate

is a general-purpose simulation timing

and performance monitor.

16       Digital gates.  Simulation definition

is written in a simple language specific

to the digital simulator.  This language

includes a CALL statement for calling

more general Pascal procedures to

simulate the gate.

32       Analog gates.  Definition consists of

the name of a Pascal procedure for

simulating the gate.

33       Analog current-mode simtype.  This type

is used internally by the analog simulator

to support current meters.

The gate's simtype defines which procedure is called

when you give a DEF command.  This command is defined

in greater detail below.

WRITE      Similar to SAVE, except the default file extension is

".text", and if no arguments are given, the current

gate is saved in a file named after the gate.

THE "DRAW" COMMAND

1.  The "DRAW" command puts you in a graphical editor mode for drawing

the visible picture of the gate.  It also allows you to place the

gate's pins and "grab box".

2.  To exit the DRAW command, press "q" or the <CNTRL>-C key.  (On

the Bobcat keyboard, this key is marked "Select".)

3.  Gate pictures are made up of many kinds of objects.  Initially,

you are set to draw the usual cyan vectors (line segments).

Tap the mouse at two locations to draw a new vector between those

points (analogous to drawing a wire in LOG).  Grab the body of

the vector to move it; grab an endpoint to change its direction

and size.  Move the vector off the grid to delete it.

4.  Tap the mouse on the color or shape name to change.  Circles and

ellipses are defined by a vector; an ellipse's diameters equal

the sides of the bounding box of the vector, and a circle's

diameter equals the larger of the two sides.  Filled ellipses

are also available.  Filled polygons are defined by four points;

to make a triangle, put two points in the same place; to make a

larger polygon, combine several triangles or quadrilaterals.

Rounded boxes are rectangles with circular arcs for corners.

"Curves" are Bezier curves defined by four points; the two

inner points govern the curve's slope at its endpoints.  Text

objects contain a string of characters of a particular size;

LOG will draw the characters at that size if possible, but

will not draw them at all if there is no font small enough.

Point-markers and box-markers are invisible placeholders which

a gate's program may use to specify the location of some

dynamic display such as an LED.

5.  Some aspects of a picture can not be edited graphically.  These

include the radius of the corners of a rounded box, and the size,

orientation and contents of a text object.  To edit the picture

textually, you can press the "e" key.  This is analogous to

the EDIT command of LOGED; it runs the text editor CAGED on a

temporary file containing the gate's definition.  See below for

a description of this format.

6.  The gate's "grab box" is a dotted yellow box that starts out at

the edges of the drawing grid.  This box defines what portion of

the picture is considered the gate's "body", for purposes of

tapping and dragging the gate in LOG.  It should generally be as

large as possible, but must not enclose any pins.  (If a pin were

inside the box, it would not be possible to draw a wire to touch

the pin.)  To move the grab box, press on any edge or corner of

the box.

7.  The pins of a gate are numbered consecutively from 1.  To create

a new pin, grab one of the numbers on the left edge of the screen

and drag it into place.  Only one pin of a given number may exist

at a time, and when the gate is saved, the pin numbers must be

contiguous (if there are "n" pins, they must be numbered 1 through

"n").  Pins are forced to multiple-of-five pixel grid lines, as are

the user's editing actions while in LOG; this makes it easier for

the user to connect exactly to the pins.

8.  To move all objects in a given rectangular area, just press the

mouse at one corner of the area (with no objects under it) and

drag out a rectangle, then move the objects into place and press

the mouse again.  This is analogous to LOG's MOVE command.  To

cancel the move, click the right button.

9.  To copy another gate's picture, press "c", then give the name

of the gate to be copied.  The current gate's picture is replaced

by the picture, pins, and grab box of that gate.  Other aspects of

the current gate (such as the name, labels, and definition) are

not affected.

10.  If you are drawing a gate which must look similar to another gate,

you can press "s" to "shadow" that gate.  Its picture will

appear dimly in the background of the editing grid.  To remove the

shadow, press "s" and enter a blank name.

11.  By default the editing grid is the exact size of a cell in the

Catalog screen or menu area.  To draw a larger gate, press "<" and

">" to zoom up or down.  If the gate is larger than the default

grid, it will be shrunk to fit in a Catalog or menu area space, but

will be shown full-size in an actual drawing.

12.  If you need more pins than there are numbers for on the screen,

press "[" and "]" to change the range of pin numbers shown.

Pin numbers may currently range up to 255.

13.  To switch to the next or previous gate in alphabetical order, press

the "n" or "p" key.  On a Bobcat, you can press the Next and Prev

keys.  This is exactly like leaving DRAW mode, typing NEXT or PREV,

and giving another DRAW command.

14.  Other keys you can press are space bar to refresh, "!" to start a

sub-shell, and "?" for help.

THE "LABEL" COMMAND

1.  To edit the text shown when the user taps on a gate in CNFG mode,

use the LABEL command.  This command runs a simple text editor.

To exit the editor, press <CNTRL>-C (or Select on Bobcats).

2.  The labels editor lets you move the cursor with the arrow keys,

delete or insert a line using the <CNTRL-F> and RETURN keys, insert

characters, or erase characters with <CNTRL-G>.  (Currently, there

is no way to recover a deleted line, or to move or copy lines except

by retyping them.)

3.  Any line without a colon (":") is displayed as-is on the screen

when the gate is configured.  To include a colon in a label, use

a double colon ("::").

4.  If a line does contain a colon, it describes an attribute.

Everything to the left of the colon is a coded string showing

the name, type, and default value of the attribute.  Everything

after the colon is shown as-is as the label for the attribute.

Blanks are ignored between parts of the attribute information.

<name> <variant> <precision> <type> <default> ':' <label>

5.  The name is optional, and must be enclosed in square brackets.

Actually, several bracketed names may be used if for some

reason the attribute must have several names.  The code for a

gate may refer to an attribute by its name, or by an integer

index ranging from 1 to the number of attributes.

6.  The type is a single capital letter.  The following types are

defined:

I     Integer.  If a precision is given, the number is

blank-padded to at least that width when displayed.

If a default is given, it becomes the initial value

of the attribute for newly created gates.  If there

is no default, the attribute is initially blank.

H     Hexadecimal integer.  The number is zero-padded to at

least the specified width; if no precision is given

but there is a default, the precision is equal to the

number of digits in the default.

R     Real number.  The precision gives the number of decimal

places shown on the number; if no precision is given

the number will be shown in floating format.  Large

and small numbers are shown in scientific notation.

U     Real number with units.  The default is of the form,

<units-string> ',' <default>

where the comma and default may be omitted if the

attribute should be blank by default.  The number is

shown followed by the units-string; large and small

numbers are shown using standard engineering notation

abbreviations.  The attribute:

[clock-freq] 2FHz,10000:Clock frequency::

will display its default as "10KHz".

F     Real number with units, scientific notation.  This is

just like "U" format except large and small numbers

are shown in scientific notation with the units-string

appended.

C     String attribute.  The precision specifies the number

of characters of length the string may have; with no

precision the string length is unlimited.  The default

may be any string of characters, unmodified except that

leading and trailing blanks are removed.

A     String attribute.  To the user, this is just like a

"C" type attribute with no precision specified.

Internally, this is stored as a variable-length

string, whereas a "C" type attribute is stored as a

fixed length string.  Precision is ignored.

B     Boolean value.  The default may be "Y" or "N" (or "T"

or "F") to specify true or false.  If the default is

blank (or "X"), the attribute is initially blank.

If the default is "T" or "F" or "X", the attribute is

displayed as "True" or "False".  If the default is

"Y" or "N" or blank, the attribute is displayed as

"Yes" or "No".

V     Variant.  This attribute represents a choice among

several named options.  The default string is actually

a sequence of comma-separated phrases.  These phrases

are numbered from 0.  The precision actually specifies

the default variant shown; with no precision, variant 0

is the default.  If the attribute should be able to be

blank, simply leave one of the phrases empty.

1 V , Yes, No, Maybe : What do you think?

This attribute takes on one of the four values blank,

Yes, No, or Maybe; it is initially Yes.

7.  A variant name may be included in any attribute, followed by a

semicolon.  The name must be one of the options for a variant

attribute that precedes this attribute in the list.  The attribute

will be invisible unless the proper variant was selected.

V Square,Circle: Kind of shape::

Square; R:Width::

Square; R:Height::

Circle; R:Radius::

8.  If the type letter is preceded by the letter 'O', the attribute is

optional and can be made blank by typing a blank line.  An attribute

with no default is always optional.  If the attribute has a default

and is not optional, then it must always have a nonblank value.  (This

is true even of string attributes.)  Entering a blank line for a

nonoptional attribute restores the default value.

THE "DEF" COMMAND

1.  The DEF command edits the simulator's definition for the gate.

What happens when you type DEF depends entirely on what simtype

you have given to the gate.  Also, for simtypes 16 and above, the

relevant simulator must have been loaded into the computer already

via a "perm" or "use" command.

2.  For simtype 0, the gate has no simulation behavior and its DEF

command does nothing.

3.  For simtype 1, the DEF command displays the signal name current

programmed into the gate, then gives you the option to change this

name if you wish.  If you leave the name blank, the gate will not

connect to any signal by default; presumably, you have set the

NAMED flag so that a signal name can be entered at run time.

4.  For simtype 7, the DEF command displays and allows you to change

the name of the Pascal procedure responsible for the gate.  This

must be of the form, "modulename_procedurename".  The writing of

Pascal coded gates is beyond the scope of this manual; refer to

Dave Gillespie's Master thesis, or examine the LOG sources for

examples.

5.  For simtype 16, you enter a simple text editor.  Use the up arrow

and down arrow keys to move, press <cntrl-K> and <cntrl-I>

to delete or insert lines. Editing functions currently not implemented

implemented are RECALL, to restore a deleted line, and left arrow

and right arrow to edit a line.  Indentation and formatting

is handled automatically.  If LOGED detects a syntax error in your

line, it converts it into a comment.  However, note that some

syntax errors are not caught but will produce code that may cause

the simulator to crash or behave strangely.  For a description of

the LOG digital gate language, see below.  Press <CNTRL>-C

or <CNTRL-D> to exit.

6.  For simtype 32, you are prompted for the name of a Pascal procedure

which takes responsibility for the gate, in much the same way as

for simtype 7.

DIGITAL GATE DEFINITION LANGUAGE

1.  Simple digital gates in LOG can be described entirely in the

modest interpreted language described here.  For more complicated

functionality, for displays and user interaction, or for access

to the gate's attributes, you must write Pascal code using a

CALL statement.  The interpreted language is usually referred to

as "LOGED language," though it is really part of the digital

simulator, not of LOGED proper.  The language is also called

Gate Description Language, or GDL.

2.  A GDL program consists of a sequence of statements, displayed one

per line.  These statements are typically either assignments to

pins or variables, or IF/ELSE/END constructions.  GDL contains no

loops, arrays, or arithmetic.  The program is executed once per

LOG time-step, from top to bottom, performing boolean operations

on values in the gate.

3.  Resources available to a GDL program are pins, notated as in "#3";

internal nodes, notated as in "##6"; and state variables, such

as "A", "P", or "V26".  A state variable is much like a Pascal

variable; an assignment to it replaces the previously assigned

value, and lasts until the next assignment.  It is a 1-bit quantity

whose value is interpreted as a boolean logic 1 or 0.  A pin or

internal node is a true LOG node; it uses a five-valued logic

(One, Zero, undriven or "None", and weak One and Zero) and must be

assigned to once per time-step.  The assigned value does not show up

on the node until the next time-step.  If the gate uses only nodes

(no state variables), the order of execution of statements within

the gate is essentially arbitrary.

4.  Pin numbers range from 1 to the number of pins on the gate.

The 16 state variables A through P are always available.  For

internal nodes and additional state variables, you can use the

INST statement described below.  All state variables are initially

Zero when the gate is created.

4.  Each GDL statement must be one of the following:

<variable> = <expression>

Assign the value of the expression to the variable.

The previous value of the variable is replaced.  This

is exactly like a variable assignment in a conventional

language.  If the value of the expression's value is

None, the variable is set to One.

<node> = <expression>

Output the value of the expression to the pin or

internal node.  If two statements (in the same or

different gates) output conflicting values to the

node in the same time-step, a conflict is registered

on the node.  Outputting None to a node has no effect.

<node> = PULLUP   or   PULLDN

Output a weak One or Zero (respectively) to the

node.  This essentially registers a default value

for the node; if no other GDL statement assigns a

strong value to the node in this time-step, the

weak default will be used.  A strong value overrides

a weak value without conflict.  A conflict will be

registered if the node is weakly pulled to One and

Zero simultaneously.

<node> '<' <expression>

Output an open-collector value to the node.  If

the expression's value is Zero, it is driven to the

node.  If the expression's value is One or None,

the node is left alone.

IF <condition> <expression>

<statements>

ELSE

<statements>

END

If the condition is satisfied, the first set of

statements are executed.  Otherwise, the second set

(if any) are executed.  The conditions available

cover all possibilities of One, Zero, and None

values for the expression:

IF x        True if x=One or None.

IFONE x     True if x=One.

IFZN x      True if x=Zero or None.

IFZERO x    True if x=Zero.

IFCONN x    True if x=One or Zero.

IFNONE x    True if x=None.

CALL <procedure-name>

Call the Pascal procedure specified.  The name has the

usual "modulename_procedurename" form.  If there are

CALL statements in a gate's procedure, those procedures

are called in order when the gate is simulated, drawn,

tapped, created, destroyed, copied, configured, etc.

INST <num-nodes> , <num-vars>

This statement, if used, must be the first statement of

the program (before even comments and blank lines).

The statement "INST 17,6" reserves 17 internal nodes,

##0 through ##16, and 6 additional state variables,

V0 through V5, for the gate.  If the number of variables

is zero, it and the comma can be omitted.

5.  GDL statements may also be blank lines or comments.  A comment

line begins with a "#" not followed by a digit.  Blank lines and

comments do have a small impact on the simulator's performance.

6.  A GDL expression is a series of "factors" joined by binary

operators.  All operators have the same precedence and are

evaluated left-to-right.  In the following list of operators,

X is any expression and Y is any factor.

x AND y    Boolean AND, OR, or exclusive OR of X and Y.  If

x OR y     either X or Y is None, the result is the other input.

x XOR y    If both are None, the result is None.

x NAND y   Equivalent to "NOT (x AND y)".

x NOR y    Equivalent to "NOT (x OR y)".

x SAME y   Both X and Y must be pins.  Equal to One if both

pins are connected to the same electrical node, or

Zero if the pins are not on the same node.

7.  A GDL factor is either an expression in parentheses, or one of

the following (X is any factor; P is any pin or internal node):

<var>      A state variable.  The result is One or Zero depending

on the current value of the variable.

<node>     A pin number or internal node.  The result is either

One, Zero, or None, depending on the value driven on

the node in the previous time-step.

ONE        The constant value, One.  This can also be written

"TRUE" or "1".

ZERO       The constant value, Zero.  Also "FALSE" or "0".

NOT x      The result is One if X is Zero, Zero if X is One, or

None if X is None.

RISE p     The result is One if the specified node is receiving

a rising transition, that is, the previous time-step

drove the pin to a One, but the time-step before that

drove it to a Zero.  Otherwise, the result is Zero.

FALL p     The result is One if the specified node is receiving

a falling (One to Zero) transition.

FIX x      The value of X, with None converted to Zero.

STRONG p   The result is One or Zero if the specified node was

driven by a strong (i.e., normal) value on the previous

time-step, or None if the node was undriven or was only

weakly driven.

8.  For examples of GDL programs, consult the many gates in the files

"log.gate" and "actel.gate".

TEXT FILE FORMAT

(This section is not yet completed.)

".GATE" FILE FORMAT

(This section is not yet completed.)

_______________________________________________________________________________

REFERENCE MATERIAL

List of commands

Configuration files

Digital simulator commands

Analog simulator commands

Single-key commands

Color names

Command-line options

X Display Preferences

LIST OF COMMANDS

1.  This is a partial list of commands that may be entered in their

full-length form (i.e., press ":", then type the command).

Many of these commands may also appear in the file "log.cnf"

which is read when LOG starts up.

2.  In this list, "ON/OFF" means either the word "ON", to turn an

option on, or "OFF", to turn an option off, or no word at all,

to "toggle" the option between on and off.

3.  Many commands have single-key equivalents.  The bindings shown

here are the defaults, which may be changed by the MACRO command.

4.  The following commands, when used in the form of single-key

commands, may be used even in the middle of drawing a wire,

selecting a rectangle, etc.:

ALTPOSN       GLOW           ON               SNAP

ARROW         GLOWSOLDER     PAGE n           TRACE

AVOID         GRID           PROBE            TRACEFILE

CLOSEFILES    HELP           QUIET            VERBOSE

CONFLICT      HOME           RESET            ZOOMDN

DOTS          INVISIBLE      RESPONSE         ZOOMUP

DUMP          INVLABEL       SHOWCONFLICTS

DUMPFILE      OFF            SIM/ONOFF/POWER

Other commands will cause any other operation in progress to

abort before the new command begins.

5.  Here is the complete list of LOG commands:

ABORT      (<CNTRL>-C) Abort the current command, mode, or screen.

ALTPOSN    ("," key.)  Scroll to an alternate location on the

page.  LOG essentially maintains two current locations

on the page, like the "mark" in the Emacs text editor;

this command exchanges them.

ARROW      Revert to the standard arrow-shaped cursor.  This undoes

any previous PROBE or GRID command.

AUTOWINDOW Toggle a new window management mode. In this mode

whenever text input into the newcrt window is necessary

the newcrt window automatically rises in front of any

windows that occlude it.  Once the window is  no longer

needed, it returns to its previous condition.

BOX        ("b" key.)  Draw a dashed box.

CAT        (Shift-"C" key.)  Switch to the Catalog screen.

CENTER     Shift the entire circuit so that it is centered in the

LOG circuit universe, i.e., so that a Home command will

home to the center of the circuit.  Note that this

actually moves the circuit elements, whereas scrolling

with the knob just changes your view on the elements.

CLEAR      Delete the entire circuit page.  Use with caution!

CLOSEFILES

Close the "dump" and "trace" files, if they are open.

These files will automatically be re-opened (for

appending) if they are again used.  The dump file is

used by the "Dump" command in the Scope.  The trace

file is used by various LOG debugging commands.

CLOSEH     Enter "close-horizontal-space" mode, similar to OPENH

except that it deletes the contents of the box you draw

and shifts other things to the left.  DANGEROUS!

CLOSEV     Enter "close-vertical-space" mode, similar to OPENV.

DANGEROUS!

CNFG       ("c" key.)  Switch gate-tapping mode to open a gate's

configuration screen when it is tapped.

COLOR  screen-color  red-value  green-value  blue-value

COLOR  color-name  screen-color

Customize the LOG color scheme.  LOG's colors are

determined in two steps.  A "palette" of up to 16

available colors is specified by the first form of

the command.  "Screen-color" is either a number from

0 to 15, or one of the names listed below.  Each of

the red, green, and blue values run from 0 (black)

to 255 (full intensity).  The second form of the

COLOR command determines which of these 16 colors

is used for various kinds of objects.  The color

names defined by LOG are listed in a later section.

Screen-colors:

0  GRAY     4  MRED     8  BLACK    12 DCYAN

1  RED      5  ORANGE   9  PINK     13 DRED

2  GREEN    6  CYAN     10 DYELLOW  14 LGRAY

3  YELLOW   7  WHITE    11 XGREEN   15 CRED

On eight-color screens, all sixteen names or numbers

may still be used; LOG combines similar colors

internally.  The effect will be that changing the

red/green/blue content of one of the above colors

may affect other colors as well on an 8-color screen.

CONFLICT n

CONFLICT ON/OFF

Turn conflict reporting on or off.  In Digital LOG,

node conflicts are reported if one gate tries to drive

a node to "1" while another is driving it to "0".

AnaLOG does not report conflicts.  Conflicts are

shown by a bright pulsating red.  If conflicts are

turned off, they are not visible on the screen.  If

an integer "n" is given, conflicts are turned on and

the number of time steps to wait before reporting a

given conflict is set to "n".  For example, if n=0,

conflicts are reported immediately.  For n=1, each

conflict must remain for at least two simulation

timesteps before it is considered interesting.

The default is conflict reporting on, with n=1.

COPY       ("/" key.)  Enter copy-area mode.

CSTOP ON/OFF

Turn conflict-stopping mode on or off.  When this mode

is on, the simulation stops (as if you had given an

OFF command) the instant any conflict is reported.

The CONFLICT command can be used to select how long a

conflict must persist before it is reported.  This

mode is off by default.

DEFINE     (Shift-"D" key.)  Display the simulator's definition for

a gate-kind, specified either on the command line or by

touching a representative gate.  Pressing shift-"D" in

the Catalog screen also displays the definition for the

gate the cursor is over.

DEL        ("d" key.)  Enter delete mode.

DOTS ON/OFF

Control dots-visible mode.  When this is on (default),

red connection dots on gates are always visible.  When

this is off, red dots are usually visible only on pins

that are not connected to wires.

DUMP       Write out a "debugging dump" to the printer.  For LOG

maintainers only.

DUMPFILE filename

Close any current dump file, then select "filename.text"

as the new dump file.  If this file already exists, new

text will be appended to it.

EXAMINE    ("e" or "x" key.)  Enter Probe mode temporarily.

The cursor returns to normal as soon as you click the

right button.  This is in contrast to the PROBE command,

which turns the Probe cursor on until you explicitly

turn it off again.

EXIT       Exit from LOG.  Also in the Misc menu, or on the Shift-Q,

Shift-Z, and Control-D keys.

EXTRACT    Delete and re-paste the entire contents of the page.  If

LOG's electrical connectivity information becomes corrupt,

this will usually fix it.

FAST       ("f" key.)  Turn on fast mode.  This happens

automatically after a certain amount of time passes with

no mouse or keyboard inputs.

GET gatename gatename gatename ...

Load the specified gates into the Catalog screen from

the gate library.  The gate names may include wildcard

characters '*' and '?'.

GLOW ON/OFF

("g" key.)  Turn glow mode on or off.

GLOWSOLDER ON/OFF

Turn glowing-solder mode on or off.  This controls

whether solder dots are visible while in GLOW mode.

It is on by default; turning glowing-solder off may

improve simulation performance in GLOW mode.

GRID       (Shift-"G" key.)  Change the cursor to a cross-hair

shape.  If the cursor is already a cross-hair, change

it back to the regular arrow cursor.

GROUP n "Name"

Set the name that appears on the top of page number "n"

of the Library screen.  The number "n" must be from 0

to 8.  If the "Name" argument is omitted, this command

instead selects which page number the Library screen

will display next time it is entered.

HELP       ("?" key.)  Display this file using the Chipmunk

Help System.

HOME       ("h" key.)  Reset the scroll and zoom amounts to the

default position on the page.

IDENTIFY   Used by the LOG-to-NTK conversion tool.

INVISIBLE ON/OFF

("i" key.)  Turn "Invisible" mode on or off.  In this

mode, wires and most gates are invisible.  However, labels,

boxes, and switch/indicator gates are visible.  Also,

operations that edit the circuit are disallowed.  Thus,

a user can switch into Invisible mode and then operate a

simulated "front panel" without confusion or fear of

changing the circuit.

INVLABEL ON/OFF

(Shift-"I" key.)  Turn "Invisible Labels" mode on or

off.  In this mode, labels and boxes are invisible.

LABEL message

("l" key.)  Create a new label on the page.  If a

"message" argument is supplied, a label with the

specified text is created on the lower-left corner of

the screen.  If no argument is supplied, LOG interactively

enters the text for the label.

LIBRARY    ("l" key or LIBR item in the Catalog screen.)  Switch

directly to the Gate Library screen.

LOAD filename

(Shift-"L" key.)  Load the specified circuit file onto

the current page, replacing anything that was on that

page before.  If no file name is specified, LOG displays

a list of files in the current directory and asks the

user to select one, or type the name.  If the file name

is '*', LOG simply asks the user to type the name.  The

shift-L key is bound to "LOAD *".

LOGNTK     Start the LOG-to-NTK conversion tool.

LOGNTK: parameter value

Set up a configuration parameter for the LOGNTK tool.

Commands of this form usually appear in the file

"/lib/log/logntk.cnf".

LPLOT: parameter value

Set up a configuration parameter for the PLOT tool.

Commands of this form usually appear in the file

"/lib/log/lplot.cnf".

MACRO  key-name  command

Associate the specified command with a key.  The key-name

may be a single letter or other character (lower-case

letters represent unshifted keys; upper-case letters

represent shifted keys); or one of the words "bs", "tab",

"sp" (space bar), or "cr" (Return or Enter); or a control

key of the form "^A" or "^["; or a three-digit decimal

ASCII code.  The "command" may be any of the commands

listed here, and may include arguments.  If you give only

a key-name, the computer displays the command that is

currently bound to the key.  MACRO with no arguments

displays a list of all current key bindings.

MAKE gate-name

Bring a gate in from the catalog by name, and put it in

the drawing area.  A shorthand for using the mouse in the

the CAT and LIBR screens.  If the gate-name contains

wildcard characters, multiple gates may be loaded into

the Catalog but only one of them will be created on the

circuit page.

MARKER ON/OFF

("m" key.)  Turn printing markers on or off.  The

printing markers are used mainly by the "PLOT" command;

they indicate the area of the circuit page which is to

be fitted into the printed page.  The printing markers

appear on the corners of the visible screen, and you

can move them by regular editing operations to bracket

any rectangular area of the page.

MESSAGE message

Display the specified message in the "message area" of

the screen.  The message area runs down the left edge

of the screen; messages are drawn in yellow overlaying

the drawing area.

MIRX       Select "mirror-in-X" gate-tapping mode.  When a gate is

tapped in this mode, it is flipped across its vertical

axis.  This works both in the menu area and in the

drawing area, although some gates (such as switches)

have special behaviors when tapped in the drawing area.

MIRY       Select "mirror-in-Y" gate-tapping mode.

MOVE       ("m" key.)  Move an area of the diagram.  This is

similar to deleting the area with DEL, then pasting

it back in a new location with PASTE.

NAME file-name

See and/or change the "current file name" for the

circuit page.  This is the file name that will be

used by default in later "SAVE" commands.

OFF        Turn simulation OFF.

ON         Turn simulation ON.

OPENH      Enter open-horizontal-area mode.  Sweep out a rectangle;

everything to the right of the lefthand edge of the

rectangle is moved to the right by the amount of the

width of the rectangle.  Wires straddling the left edge

are stretched appropriately.  BEWARE:  This command

may corrupt the LOG data structures if used in such a

way as to break or make electrical connections in the

circuit.  Unless you really know what you are doing,

use the cut-and-paste method of moving things.

OPENV      Enter open-vertical-area mode, similar to OPENH.

DANGEROUS!

PAGE n     Switch the display to circuit page "n", from 1 to 9.

"n" may also be "+" or "-", to switch to the next or

previous numbered page.

PASTE      Enter Paste mode.

PLOT       Run the circuit plotting tool.

POPUP  menu  position  kind  command  message

Change one of the locations in the pop-up menus.  "Menu"

is a number that selects which menu (1=Frills, 2=Editing,

3=Cursor, 4=Misc); "position" is an integer from 1 to 8

that identifies which spot in the menu:   [ 1  3  5  7 ]

[ 2  4  6  8 ]

"Kind" is normally zero; "command" is the command (from

this list) that is to be executed if the menu is selected,

enclosed in "quotes" if it contains spaces; and "message"

is the wording for that menu element.  If "message" begins

with '*' or '#', it will be colored red or light blue,

otherwise it will be green.

PROBE      ("." key.)  Change to the Probe cursor.  If already

using the Probe cursor, change back to an arrow.  In

Probe mode, holding the cursor near a wire or gate pin

displays the value on that electrical node, in a

simulator-dependent way.  Holding the cursor near a

gate in the drawing or menu areas displays the name

of the gate on the screen.  Tapping a gate or node

opens it for configuring, if it has anything to

configure.  (Neither AnaLOG nor Digital LOG nodes

are configurable, but many gates are.)  Tapping and

configuring a gate in the menu area sets the defaults

for newly created gates pulled out of that menu slot.

Finally, tapping out in the drawing area draws a

yardstick.  See also the EXAMINE command (on the

"e" and "x" keys) which selects a temporary

Probe mode.

QUIET      ("q" key.)  Turn sound prompting on or off.

READ  filename

Read in a circuit from a file.  This is similar to the

LOAD command, but it rebuilds the circuit's connectivity

information as it loads, rather than simply reading it

from the file.  As a result, READ is much slower than

LOAD, but it is far more robust in case the file is

corrupt or out of date.  For example, if you run LOGED

and add more pins to a gate, then try to LOAD a circuit

diagram containing that gate, LOG may crash since the

connectivity recorded in the file is out of date.  But

the READ command will read the file with no problems.

In general, use the fast LOAD command unless you have

a good reason not to.

REFRESH    (Space bar.)  Refresh the screen.

RESET      (Shift-"R" key.)  Send a Reset signal to the simulator(s).

This resets simulation time back to zero, and has other

simulator-dependent effects.  For AnaLOG, it resets all

voltages to their initial values.  For Digital LOG,

nothing happens except that the TO/FROM signal "Reset",

if used, gets a brief "1" pulse.

RESPONSE  min  max  rate

Set response-time parameters.  With no arguments,

displays current parameters.  Initial values are

2, 50, and 35, respectively.  During editing, LOG

tries to check the mouse and keyboard at least every

"min" centi-seconds.  If the user stops editing,

this rate slowly increases until the maximum of

"max" centi-seconds.  The rate of increase is

one unit per "rate" centi-seconds of idle time.

ROT        Select "Rotate by 90 degrees" gate-tapping mode.  When

a gate is tapped in this mode, it rotates 90 degrees

counter-clockwise.  This works both in the menu area and

in the drawing area, although some gates (such as

switches) have special behaviors when tapped in the

drawing area.

SAVE filename

Save circuit page(s).  If a name is given, saves the

current page in "name.LGF".  If the name is "*", prompts

the user to enter the name to save.  If no name is

given, re-saves all circuit pages which have been

since last saved or loaded.  Shift-"S" is bound

to "SAVE *".

SCOPE      ("s" key.)  Switches to the Scope screen.

SHELL shell-command    ( or:  !shell-command )

(Also, "!" as a single-key command.)  Start a sub-Shell.

If a shell-command is provided the shell executes this

one command and returns.  If no shell-command is provided,

you get an interactive sub-shell.  Type quit or press

control-D to return from the sub-shell.

SHOWCONFLICTS

("k" key.)  Turn "ShowConflicts" mode on or off (toggling).

Default is off.  In this mode, wires and most gates are

invisible, similar to invisible mode.

However, nodes that are in conflict remain visible to

aid their identification.

SIM (or ONOFF or POWER)

Turn simulation ON or OFF (toggling).

SNAP       ("$" key.)  Turn "snap-to-grid" mode on or off.  Default

is off.  In this mode, the cursor arrow always jumps

to the nearest spot on the LOG editing grid rather

than moving smoothly.

STATUS     Enter the "Status" display.  Use the knob or arrow

keys to switch among various pages.  A page-name may

also be given with the STATUS command:

LOG      General system status

MEMORY   Memory usage status

MACRO    Keyboard mappings

16       Digital simulator

32       Analog simulator

LPLOT    Plotting tool

LOGNTK   LOG-to-NTK tool

STEP n     Turn simulation OFF, if necessary, then step the

simulation by one time-step.  If an integer "n" is

provided, executes "n" time-steps before stopping.

TAPMODE    (menu selection, or shift-"M" key.)  Select the next

gate-tapping mode, in order:  ROT, MIRX, MIRY, CNFG.

TOOL name  Send a "select" command to the specified tool.  Various

tools respond to this in various ways; the default is

to ignore the signal.  (The Digital LOG simulator, tool

name "16", responds by putting up a configuration

screen of digital-LOG parameters.)  If a TOOL command

is given with no "name" argument, it puts up a screen of

available tool names and "selects" whatever tool the

user chooses with the mouse.  The shift-"T" key calls

up an interactive TOOL command.

TRACE      Turn the debugging "wallpaper" mode on or off.  Please

don't do this unless a LOG maintainer is there.  Trace

output goes to the "Trace" file, which by default is an

appropriate printer.

TRACEFILE filename

Close any current trace file, then select "filename.text"

as the new trace file.  If this file already exists, new

text will be appended to it.

VERBOSE    ("v" key.)  Turn informational messages on or off.  These

messages are mostly of the "Glow mode is now OFF"

variety.

VMESSAGE   Similar to MESSAGE, but works only if in

Verbose mode.

YARDSTICK  Draw a "yardstick", useful for measuring the spacing of

objects in your diagram.

ZOOMDN     Zoom down, to view more of the circuit page.

ZOOMUP     Zoom up, to view part of the circuit in greater

detail.

CONFIGURATION FILES

1.  LOG reads the file "log.cnf" every time it starts up.  If you do

not have one in the current directory, it uses "/lib/log/log.cnf".

You can give the "-c" command-line switch to specify your own

configuration file.  The "use log" script actually loads the

program "diglog" into the computer and aliases the "log" command

to "diglog -cdlog", causing "dlog.cnf" (in the current directory

or in /lib/log) to be used.  Similarly, "use analog" loads the

program "analog" and aliases "log" to "analog -calog".

2.  To create your own configuration file, the best approach is to

make a file named "log.cnf", "alog.cnf", or "dlog.cnf", whichever

is appropriate, in your current directory.  In that file, put a

line something like "include /lib/log/alog.cnf", followed by any

additional commands to override the system defaults.  Do not

copy the system CNF files if you can avoid it; these files are

subject to change at any time.

3.  For example, your CNF file might look like this:

{ A comment saying what this is used for }

include /lib/log/dlog.cnf     { Inherit from Digital LOG }

gates + ~me/log/mygates       { Add on my own gates file }

menu a b c d e vdd gnd        { Choose my own menu gates }

glow on                       { Turn on "glow" mode }

In this example, the "+" in the GATES command means to add the

name(s) to the existing list of gates files.  Since the MENU

command does not have a "+", it replaces the list from the

other file.  This notation works with the GATES, GET, GETGROUP,

and MENU commands in configuration files.

4.  The following ":" commands may also be used in a "log.cnf"

configuration file:

AVOID         GLOWSOLDER    OFF           QUIET

COLOR         GROUP         ON            RESPONSE

CONFLICT      INVISIBLE     SIM           SNAP

DOTS          INVLABEL      POPUP         VERBOSE

GLOW          MACRO         PROBE

5.  The following additional commands are allowed in configuration

files:

BOBCAT  command-line

CHIPMUNK  command-line

Process the command only if LOG is being run on the

specified kind of computer.  The "command-line" here

is any of the commands listed in this section; in other

words, the words BOBCAT and CHIPMUNK are prefixes that

can be added to any command in a CNF file.

COMMAND  toolname  command command command ...

Register that the listed commands are handled by the

indicated tool.  If a command is given and its owning

tool has not been initialized, that tool will be

initialized.

DO command-line

Add the command-line to a list of ":" commands to be

done as soon as LOG is started.  These commands are

executed as if typed from the keyboard, i.e., they

may be commands which are ordinarily forbidden in

CNF files.  The "DO" commands are executed in the

order they originally appeared in the CNF files.

GATES filename filename filename ...

Register the specified ".gate" files into the LOG

gate library.  Files are searched for a given gate

starting at the first filename of the first GATES

command in the CNF file.  Ordinarily, the presence

of the first GATES command in a given CNF file

overrides all GATES commands in previously read

CNF files (i.e., INCLUDE files).  If the first

word after GATES is "+", the listed filenames will

instead be added to the existing list of gates

files.  If no explicit directory is given, LOG

looks first in the current directory, then in

the "home" directory, then in /lib/log.  If the

gates file is not found, it is ignored.

GET gate gate gate ...

Load the specified gates into the gate catalog.  The

gate names may include wildcard characters "*" and "?",

and may also specify one or more group numbers from

0 to 8:  "GET A*;34" gets all gates whose names begin

with A, which are found on pages 3 or 4 of the Library.

If this is the first GET command in its CNF file, it

overrides any previous GET commands from other files

unless GET is followed by the word "+".  (The obsolete

word LOAD is accepted as a synonym for GET.)

GETGROUP gate gate gate ...

Load the specified group of gates into the gate catalog.

This like a normal GET command, except the gates are

guaranteed to be placed together (provided there's

room), and a box is drawn around them on the catalog

screen.

HELP filename

Name of a ".help" file to be used when the user

asks for help.

HOME directory-name

Specify a directory to be used as the "home"

directory.  If a CNF file, gates file, help file,

or news file is not found in the current directory,

LOG looks next in this "home" directory, then

finally in "/lib/log".  The default "home" directory

is "~/log", that is, the "log" subdirectory of the

user's home directory.

INCLUDE cnf-file

Read commands from the specified ".cnf" file, then

resume reading from this file.  Include files may

be nested.

MENU gate gate gate ...

Load the specified gates into the menu area.  If

there are more gate names than menu slots, only the

first few names are used.  The first word may be

"+" to add the specified gates to a list from a

previous CNF file.

NEWS file-name

Name of a ".text" file whose contents are displayed

when LOG begins.

SIGNALS number

(or NODES.)  Specifies the maximum number of TO/FROM

signal names that can exist at one time during the

LOG session.  Default is 200.

TABLET number

Set the HP-IB address of the graphics tablet to be

used.  Default is 0, which means look for the tablet

on the HP-HIL or at the standard address.

TOOL tool-name "full-name"

Register a tool name so that the run-time TOOL command

will display its name.  (Ordinarily, tools are

automatically registered as soon as they are needed.)

UNDO       Delete all "DO" commands registered up to this point.

tool-name: tool-command

This directs a command to a particular LOG "tool".

The standard tool names are:

16       Digital simulator

32       Analog simulator

LPLOT    Plotting tool

LOGNTK   LOG-to-NTK tool

DIGITAL SIMULATOR COMMANDS

1.  The following additional ":" commands are available with the

Digital LOG simulator.  Except for STABILIZE they may be used

in "log.cnf" files by preceding them with the word "16:", as

in "16: stabdelay 2.5".

DIGON      Turn digital simulation ON.  This is the normal case.

DIGOFF     Turn digital simulation OFF.  This command is similar

the regular OFF command, except only the digital

simulator is affected.  If several simulators are

being run at once (say, digital and analog), the

other simulators will proceed unaffected.

DIGONOFF ON/OFF

Turn digital simulation ON or OFF.

STABDELAY time

Set amount of time to wait for circuit stabilization

(see STABILIZE) to "time" seconds.  Default is 5.0.

With no argument, the command shows the current value.

STABILIZE  Attempt to stabilize an oscillating circuit.  Because

all digital gates have identical, exact propagation

delays, oscillations can develop in feedback circuits

that would never arise in a real circuit, where the

propagation delays are not so exactly balanced.  The

STABILIZE command randomizes the propagation delays of

gates in order to kill such oscillations.  Of course,

a circuit that really is oscillating will continue

to do so.  The "stabilization" effect wears off after

about five seconds, usually plenty of time.

DIGSTEP time-step

Set the amount of simulated time per digital timestep.

Since every digital time step is the same, this value

normally does matter unless you are using the digital

simulator along with another simulator which has a more

concrete idea of time (such as AnaLOG).  The default

value is "10ns", i.e., 1E-8 seconds.  The value may

be specified in engineering or real-number notation.

With no argument, the command shows the current value.

ANALOG SIMULATOR COMMANDS

1.  The following additional ":" commands are available with the

AnaLOG simulator.

DIM ON/OFF

Turn dim-unconnected-gates mode on or off.

ERASE      Same as touching Erase on the "Scoreboard" gate.

If you are selecting Set and Erase often, you might

want to use the MACRO command to put SET and ERASE on

a pair of keys, for convenience.

EXACT      Same as touching Exact on the "Scoreboard" gate.

NEWMODEL   Set non-saving device-model mode, in which modeling

parameters for transistors, and simulation constants

such as "Vdd", are reset to their default values

rather than being loaded from each circuit file.

OLDMODEL   Set the default device-model mode, in which modeling

parameters for transistors, and simulation constants

such as "Vdd", are saved in circuit files along with

the transistor and "Scoreboard" (NUMBERS) gates.

PROBEPREC digits

Set the number of decimal digits for voltages displayed

by the "probe" cursor.  Default is 3.

RELAXED    Same as touching Relaxed on the "Scoreboard" gate.

SET        Same as touching Erase on the "Scoreboard" gate.

2.  Also, a great many AnaLOG parameters may be set in ".CNF" files.

These commands all begin with the prefix "32:".  Defaults appear

in the file "models.cnf" in the analog /lib directory -- ask

your local maintainer for details

SINGLE-KEY COMMANDS

Many commands also appear in single-key forms.  In this list, lower-case

letters represent unshifted keys, and capital letters are shifted keys.

:         Enter a full-line command.

space     Refresh the screen:  REFRESH.

knob      (or arrow keys.)  Scroll across the page.

<CNTRL>-C Abort the current command or mode.

1-9       (Digits.)  Switch to a new page.  Same as PAGE 1 to PAGE 9.

!         Shell escape:  SHELL.

*         Draw the contents of the Paste buffer:  PASTE.

,         (Comma.)  Switch to alternate viewing position:  ALTPOSN.

.         (Period.)  Enter or leave PROBE mode.

/         Copy an object or area into the Paste buffer:  COPY.

<         Zoom down, to see more of the diagram at once:  ZOOMDN.

>         Zoom up, to focus on a smaller area of the diagram:  ZOOMUP.

?         Display this help file.

+         (also NEXT on Bobcats.)  Switch to next higher page number.

-         (also PREV on Bobcats.)  Switch to next lower page number.

b         Draw a dashed box:  BOX.

c         Switch to "CNFG" gate-tapping mode:  CNFG.

C         Enter the Catalog screen:  CAT.

d         Delete objects:  DEL.

D         Show simulator's definition for a gate:  DEFINE.

e         Enter temporary PROBE mode:  EXAMINE.

f         Turn on FAST mode.

g         Turn GLOW mode on or off.

G         Turn GRID (cross-hair cursor) mode on or off.

h         Return to home (unscrolled) position:  HOME.

i         Turn INVISIBLE mode on or off.

I         Turn INVLABEL (invisible labels) mode on or off.

k         Turn SHOWCONFLICTS mode on or off.

l         Draw a label:  LABEL.

L         Load a new circuit page:  LOAD.

m         Move an object or area:  MOVE.

M         Select the next gate-tapping mode:  TAPMODE.

$         Turn SNAP-to-grid mode on or off.

o         Turn simulation on or off:  ONOFF.

p         Enter the circuit-plotting tool:  PLOT.

r         Switch to "ROT" gate-tapping mode:  ROT.

R or 0    Reset the simulators:  RESET.

q         Turn QUIET mode on or off.

Q or Z    (or Control-D.)  Exit from the LOG system:  EXIT.

s         Enter the Scope screen:  SCOPE.

S         Save the current page to a file:  SAVE.

t         Run simulation for one time-step, then stop:  STEP.

T         Enter the Tool screen:  TOOL.

v         Turn VERBOSE mode on or off.

x         Enter temporary PROBE mode:  EXAMINE.

y         Draw a YARDSTICK.

COLOR NAMES

1.  The following list is the complete set of color-names defined so

far in the LOG system.  See the COLOR command for details

on how to reconfigure these color names.

2.  After each color-name is its default color, then a description of

what screen objects that color-name governs.

BACKGR     Gray              Screen background color.

BASELINE   Cyan              Color of line above menu area.

BLUEWORD   Cyan              Light blue text in pop-up menus.

CATALOGBOX Dim yellow        Gate grouping box in Catalog screen.

CATGATE    Cyan              Color of gates in Catalog screen.

CHART      Yellow            Default color of traces in Scope.

CONFLICT   C-Red             Color of "signal-conflict" in a wire.

CURSOR     White             Regular cursor color.

DASHBOX    Dim yellow        Color of dashed boxes in circuits.

DEFINEBACK Black             DEFINE command's box background.

DEFINEBOX  Cyan              DEFINE command's box outline.

DEFINETEXT Green             DEFINE command's text color.

DIMGATE    Dim cyan          Color of "dimmed" gates in circuits.

DIVISION   White             Color of grid dots in Scope.

GATE       Cyan              Normal color of gates in circuits.

GATEBLACK  Black             Black parts of gate pictures.

GATEGREEN  Green             Green parts of gate pictures.

GATEORANGE Orange            Orange parts of gate pictures.

GATEPIN    Red               Color of gate connection dots.

GATERED    Red               Red parts of gate pictures.

GATEWHITE  White             White parts of gate pictures.

GATEYELLOW Yellow            Yellow parts of gate pictures.

GLOW1      Black             AnaLOG glow color for <= 0 volts.

GLOW2      Dim red           AnaLOG glow color for 0-15

GLOW3      Medium red        AnaLOG glow color for 15-50

GLOW4      Red               AnaLOG glow color for 50-85

GLOW5      Pink              AnaLOG glow color for 85-100

GLOW6      White             AnaLOG glow color for >= Vdd.

IDENTIFY   Red               Color of identified things in LOGNTK.

IMETER     Orange            AnaLOG current meter readout.

INSTANCE   Dim yellow        Name shown in Digital instance gates.

KINDGATE   Green             Color of gates in menu area.

LABELTEXT  Dim yellow        Color of circuit labels.

LIMITON    Red               AnaLOG current-limit indicator.

MARKER     Pink              Color of printing markers.

MENUWORD   Green             Color of text in menu area.

MESSAGE    Yellow            Informative messages in drawing area.

OFFLED     Black             Digital LOG logic "0" signal.

ONLED      Red               Digital LOG logic "1" signal.

PAGE1      Green             Color of the word "PAGE".

PAGE2      Yellow            Color of current page number.

PAGE3      Green             Color of the word "OF".

PAGE4      Yellow            Color of maximum page number.

PEN1       White             Color for plotter pens #1-6.

PEN2       Red               . These colors are used by the PLOT

PEN3       Green             . command for circuit and data

PEN4       Orange            . plotting.

PEN5       Cyan              .

PEN6       Yellow            .

POPUPBOX   White             Outline of pop-up menus.

POPUPWORD  Green             Text in pop-up menus.

POPUPSEL   Pink              Highlighted text in pop-up menus.

PROBE      Dim yellow        AnaLOG probe-mode display color.

REDWORD    Red               Red text in pop-up menus.

SCROLL     White             Color of scroll-indication lines.

SELECT     White             Color of area-selection rectangle.

SELWORD    Yellow            Highlighted words in menu area.

SIGNAL     Pink              Color of TO/FROM signal names.

SOLDER     Green             Color of solder dots on wires.

SWITCHOFF  Light gray        AnaLOG switch "off" color.

SWITCHON   Light gray        AnaLOG switch "on" color.

TEMPLATE   Pink              Name shown in Digital template gates.

TIME       Dim yellow        Color of TIME gate display

VMETER     Dim yellow        AnaLOG voltage meter readout.

WIRE       Green             Color of non-glowing wires.

XWIRE      X-Green           Color of wires being drawn.

COMMAND-LINE OPTIONS

These are all the command-line options log currently understands:

-x displayname     : This option specifies the screen where the

newcrt and mylib windows will appear. For

example, the command analog -x foo.school.edu:0.0

results in the newcrt and mylib windows

appearing on the console of the machine

foo.school.edu. An alternative to setting the

DISPLAY environment variable in the launch shell.

-c filename        : Specifies a .cnf configuration file.

X DISPLAY PREFERENCES

Log looks in your .Xresources file for geometry information.  If set

properly, this allows windows of a certain size to be automatically

placed at a particular place at every invocation of log; manual

placement of windows is no longer necessary. This feature was added by

Jim Clark at Harvard. Adding these lines to your .Xresources file:

mylib.geometry: =1150x550+0+0

newcrt.geometry: +510+586

will result in good placement and sizing for the color monitors

shipped with SPARC IPC's; for other screens, charge the numbers after

"+" signs for new positions, and change the numbers between the "x"

for modify the size of the mylib window. The newcrt window cannot be

modified; log expects it to be a fixed size.

Psys now senses if a display screen is black and white, and tries to

use stipple instead of colors. However, there are times where you want

to force the tools to display in black and white, even if the display

screen is color. These times include using an OS/2 based color PC as an

X terminal for the analog tools, because of present incompatibilities

between PC graphics cards and analog. To force a black and white

display, add this line to your .Xresources file:

mylib.color:  black_and_white

and then restart your X server. Black and white support was provided

by Tor Sverre Lande (Oslo), and the .Xresources flag was added by Mike

Godfrey (Stanford).

Color functions can also be set via an environment variable,

LOG_COLOR.

Chipmunk tools currently support monochrome, 8-bit, 16-bit, and 24-bit

displays.  The 16-bit and 24-bit display modes are only known to work

on the PC architecture (X11 under Linux and OS/2).

By default, the Chipmunk tools select a color mode using these rules:

If Pseudo-Color mode is supported with a depth of at least 8 bits, the

tool uses 8-bit mode.

If Pseudo-Color mode is not supported but TrueColor mode is supported,

the tool uses 24-bit depth if available, or 16-bit depth if 24-bit

depth is not available.

If all else fails, monochrome mode is selected.

The user can force the color mode of a Chipmunk tool by setting the

environment variable LOG_COLOR to be the string bw or 8bit or 16bit or

24bit.

(End of LOG documentation.)